Risk Report for the Mokelumne River

| Assessment Name: | Clover II |
| --- | --- |
| Assessment Date: | 9/17/2007 |
| Total Distance (ch): | 255 |
| Total Risk: | <1% |

Description

Clover II fire

| Term File Information |  |
| --- | --- |
| Season Start Date: | 5/1/2007 |
| Alpha Value: | 3.2 |
| Beta Value: | 0.0066 |
| File name: |  |

Summary of Results by Time Period

| Period # | Start Date | Name |  | Risk |
| --- | --- | --- | --- | --- |
| 1 | 5/1/2007 | Clover II |  | <1% |

Segment Descriptions

| Seg # | Map ID | Veg Type | Fuel Model | Length (ch) | Slope % | Aspect | Elev | Shelter | Shade | Wind Vector | Spread Dir |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A | SH | 5 | 106 | 3 | S | L | U | U | 180 | 180 |
| 2 | B | SH | 4 | 24 | 10 | S | L | U | U | 180 | 180 |
| 3 | C | TL | 8 | 12 | 20 | S | L | S | S | 180 | 180 |
| 4 | D | SH | 4 | 1 | 33 | S | L | U | U | 180 | 180 |
| 5 | E | TL | 8 | 42 | 20 | S | L | S | S | 180 | 180 |
| 6 | F | SH | 4 | 18 | 27 | S | L | U | U | 180 | 180 |
| 7 | G | TL | 8 | 4 | 16 | S | L | S | S | 180 | 180 |
| 8 | H | SH | 4 | 18 | 53 | S | L | U | U | 180 | 180 |
| 9 | I | TL | 8 | 15 | 48 | S | L | S | S | 180 | 180 |
| 10 | I | TL | 10 | 15 | 32 | S | L | S | S | 180 | 180 |

Risk by Segment

| Segment Number | Chains from Current Fire Position | Probability of Fire Reaching End of Segment Before a Season Ending Event |
| --- | --- | --- |
| 1 | 106 | 46 % |
| 2 | 130 | 38 % |
| 3 | 142 | 27 % |
| 4 | 143 | 26 % |
| 5 | 185 | 5 % |
| 6 | 203 | 4 % |
| 7 | 207 | 3 % |
| 8 | 225 | 2 % |
| 9 | 240 | 1 % |
| 10 | 255 | < 1 % |

Time Period Details

| Time Period: | 1 |
| --- | --- |
| Start Date: | 5/1/2007 |
| Period Name | Clover II |

| Weather Observations (days): | 6151 |
| --- | --- |
| Aligned Observations (days): | 1200 |
| Aligned Percent: | 20% |

Conditions by Severity Level

|  | Low | Mod | High | Ext |
| --- | --- | --- | --- | --- |
| Probability | 15 | 75 | 7 | 3 |
| Uncorrected 1 hour FM % | 7 | 6 | 6 | 5 |
| 10 hour Fuel Moisture % | 11 | 7 | 8 | 11 |
| 100 hour FM % | 15 | 13 | 13 | 12 |
| Herbaceous FM % | 129 | 87 | 62 | 51 |
| Woody FM % | 133 | 109 | 96 | 93 |
| 20' wind speed | 3 | 6 | 14 | 18 |
| 1000 hour FM % | 19 | 16 | 16 | 14 |

Data by Time Period, Severity Level, and Segment

| Seg # | Map ID | Moisture Correction |  |  |  | Wind Adjustment |  |  |  | Flame Length |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Low | Mod | High | Ext | Low | Mod | High | Ext | Low | Mod | High | Ext |
| 1 | A | 0 |  |  |  | 0.4 |  |  |  | 1.0 |  |  |  |
| 2 | B | 0 |  |  |  | 0.6 |  |  |  | 9.2 |  |  |  |
| 3 | C | 3 | 3 | 3 | 3 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.4 | 0.6 | 0.8 |
| 4 | D | 1 |  |  |  | 0.6 |  |  |  | 6.1 |  |  |  |
| 5 | E | 3 | 3 | 3 | 3 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.4 | 0.6 | 0.8 |
| 6 | F | 0 |  |  |  | 0.6 |  |  |  | 7.5 |  |  |  |
| 7 | G | 3 | 3 | 3 | 3 | 0.2 | 0.2 | 0.2 | 0.2 | 0.3 | 0.4 | 0.7 | 0.8 |
| 8 | H | 1 |  |  |  | 0.6 |  |  |  | 4.0 |  |  |  |
| 9 | I | 3 | 3 | 3 | 3 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.3 | 0.5 |
| 10 | I | 3 | 3 | 3 | 3 | 0.2 | 0.2 | 0.2 | 0.2 | 1.0 | 1.1 | 2.8 | 3.5 |

| Seg # | Map ID | Hourly Rate of Spread (ch) |  |  |  |  | Burn Period (hrs/day) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Low | Mod | High | Ext | Unsel | Low | Mod | High | Ext | Unsel |
| 1 | A | 1.3 | 3 M | 4 M | 5 M | 1 | 3 | 4 | 5 | 6 | 1 |
| 2 | B | 16 | 3 M | 4 M | 5 M | 1 | 3 | 4 | 5 | 6 | 1 |
| 3 | C | 0.1 | 0.3 | 0.7 | 1.0 | 1 | 3 | 4 | 5 | 6 | 1 |
| 4 | D | 6 | 3 M | 4 M | 5 M | 1 | 3 | 4 | 5 | 6 | 1 |
| 5 | E | 0.1 | 0.3 | 0.7 | 1.0 | 1 | 3 | 4 | 5 | 6 | 1 |
| 6 | F | 10 | 3 M | 4 M | 5 M | 1 | 3 | 4 | 5 | 6 | 1 |
| 7 | G | 0.1 | 0.3 | 0.7 | 1.1 | 1 | 3 | 4 | 5 | 6 | 1 |
| 8 | H | 2.5 | 3 M | 4 M | 5 M | 1 | 3 | 4 | 5 | 6 | 1 |
| 9 | I | 0.1 | 0.1 | 0.1 | 0.5 | 1 | 3 | 4 | 5 | 6 | 1 |
| 10 | I | 0.3 | 0.3 | 2.5 | 4.0 | 1 | 3 | 4 | 5 | 6 | 1 |

| Seg # | Map ID | Daily Spread (ch) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Low | Mod | High | Ext | Unsel |
| 1 | A | 3.9 | 12 | 20 | 30 | 1 |
| 2 | B | 48 | 12 | 20 | 30 | 1 |
| 3 | C | 0.3 | 1.2 | 3.5 | 6 | 1 |
| 4 | D | 18 | 12 | 20 | 30 | 1 |
| 5 | E | 0.3 | 1.2 | 3.5 | 6 | 1 |
| 6 | F | 30 | 12 | 20 | 30 | 1 |
| 7 | G | 0.3 | 1.2 | 3.5 | 6.6 | 1 |
| 8 | H | 7.5 | 12 | 20 | 30 | 1 |
| 9 | I | 0.3 | 0.4 | 0.5 | 3 | 1 |
| 10 | I | 0.9 | 1.2 | 12.5 | 24 | 1 |