| INCIDENT RADIO COMMUNICATIONS PLAN |  |  | Incident Name: CANYON COMPLEX |  |  | Date/Time Prepared: 6/25/07 2100 |  | Operational Period Date/Time: 6/26/08 0700 to 1900 Hrs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | USE HUMAN REPEATER WHEN NECESSARY ! ! ! |  |  |  |  |
| Ch # | Function | Channel Name/Trunked Radio System Talkgroup | Assignment | RX Freq N or W | RX Tone/NAC | TX Freq N or W | Tx Tone/NAC | Mode | Remarks |
| 1 | # Tactical | NIFC Tac 1 | Div A, Crest/Spanish South Group/Sloat Staging | 168.0500 N |  | 168.0500 N |  | A |  |
| 2 | Tactical | NIFC Tac 2 | Div B, Big, North Group | 168.2000 N |  | 168.2000 N |  | A |  |
| 3 | Tactical | NIFC Tac 3 | Div C/Z, Pit/Murphy | 168.6000 N |  | 168.6000 N |  | A |  |
| 4 | Tactical | R 5 Tac 4 | I/A, Granite/Rock/Quarry/Island | 173.9125 N |  | 173.9125 N |  | A |  |
| 5 | AIR/GROUND | A/G | All Divs | 172.4625 N |  | 172.4625 N |  | A |  |
| 6 | CMD AIR/GROUND | CMD A/G | OPS Staff | 172.6250 N |  | 172.6250 N |  | A | Air, Ops, Branches, Div Sups ONLY |
| 7 | COMMAND | Mt. Hough | All Divs | 168.0750 N |  | 170.4250 N |  | A | NIFC C3 RPTR – LINKED |
| 8 | COMMAND | Claremont | All Divs | 162.9625 N |  | 171.7875 N |  | A | NIFC C7 RPTR – LINKED |
| 9 | COMMAND | Mills Pk | All Divs | 170.4125 N |  | 165.9625 N |  | A | NIFC C10 RPTR – LINKED |
| 10 | COMMAND | Red Hill | All Divs | 166.5625 N |  | 172.8125 N |  | A | NIFC C18 RPTR - LINKED |
| 11 | FOREST NET | Mt. Hough | All Divs | 170.5500 N |  | 169.9000 N |  | A |  |
| 12 |  |  |  |  |  |  |  | A |  |
| 13 | R 5 Air/Ground | R 5 Air/Ground |  | 170.0000 N |  | 170.0000 N |  | A |  |
| 14 | Air Guard |  |  | 168.6250 N |  | 168.6250 N | 110.9 | A | Ground to Air Emergency Only |
| 15 |  |  |  |  |  |  |  |  |  |
| 16 | Air Guard |  |  | 168.6250 N |  | 168.6250 N | 110.9 | A | Ground to Air Emergency Only |
| 5. Prepared by (Communications Unit) Kevin Harper/ Clay Flad COML |  |  |  |  | Incident Location County Plumas State CA Latitude N 39 56.257 Longitude W 120 56.408 |  |  |  |  |

The convention calls for frequency lists to show four digits after the decimal place, followed by either an “N” or a “W”, depending on whether             6/25/08

the frequency is narrow or wide band.   Mode refers to either “A” or “D” indicating analog or digital (Project 25)