| FIRE BEHAVIOR FORECAST |  |
| --- | --- |
| FORECAST NUMBER 17 | TYPE OF FIRE SURFACE |
| FIRE NAME BTU LIGHTNING COMPLEX | OPERATIONAL PERIOD JULY 14, 2008 |
| DATE ISSUED JULY 13, 2008 | TIME ISSUED 1900 |
| UNIT BUTTE UNIT, PLUMAS AND LASSEN NATIONAL FOREST | SIGNED |
| INPUTS |  |
| WEATHER SUMMARY SEASONAL TEMPERATURES AND RELATIVE HUMIDITY. GOOD HUMIDITY RECOVERY AT NIGHT. SEE INCIDENT METEOROLOGIST FORECAST |  |
| OUTPUTS FIRE BEHAVIOR |  |
| GENERAL INPUTS FINE DEAD FUEL MOISTURE 6-7% UNDER SMOKE SHADING--- PROBABILITY OF IGNITION= 56% MAXIMUM SPOTTING DISTANCE 0.2 MILES. CRITICAL WIND SPEED FOR CROWN TRANSITION 13 MPH AT TREE TOP LEVEL. A SMALL POSSIBILITY OF CONVECTION OVER THE SIERRA VALLEY OR THE CREST COULD CAUSE STRONG GUSTY WINDS WITH LITTLE WARNING. SPOTTING AND CROWNING WILL RESULT. ANY HOT AREA IS A DANGER IF THIS OCCURS. REBRN IS POSSIBLE UNDER THIS SCENARIO. |  |
| SPECIFIC CONCOW BRANCH SMOLDERING HEAVY AND DOZER PILES SHOULD BE THE FOCUS OF ACTIVITY WITH THE CHANCE OF HOTTER WEATHER LATER THIS WEEK OR NEXT WEEK. THE SOUTH END OF THE FIRE MUST STAND A WIND TEST SOME TIME, HOPEFULLY LATER THAN SOONER. SCORCHED CANOPY WITHIN 300’ OF THE LINE WILL NEED COMPLETE OVERHAUL AS WELL. WEST BRANCH EXPECT SURFACE FIRE BEHAVIOR WITH RATE OF SPREAD 10-14 FT/MIN AND 2-6 FT FLAMES AFTER 1200. NORTH FACING SLOPES IN FLEA VALLEY HAVE HEAVIEST FUELS AND WILL HOLD HEAT. LONG BRANCH IN ROCK CREEK THE FIRE IS MAKING FISH-HOOK RUNS UP A WEST FACING SLOPE FLANKING TO THE NORTH. EXPECT SURFACE FIRE BEHAVIOR WITH RATE OF SPREAD 2-10 FT/MIN AND 5-6 FT FLAMES BETWEEN 1200 AND 1600. SINGLE TREE TORCHING IS POSSIBLE WITH STRONGER WIND GUSTS AND/OR FUEL JACKPOTS UNDER CROWNS. LIGHTER FUELS IN THE STORRIE BURN WILL SLOW FIRE SPREAD TO THE NORTH, BUT MOVEMENT TO THE WEST WILL CONTINUE AFTER 1100. |  |
| AIR OPERATIONS: - SMOKE ISSUES CONTINUE. SOME LIFTING BETWEEN 1100 AND 1600 |  |
| SAFETY - HAZARD TREES ARE THE BIGGEST DANGER TO YOUR LIFE ON THIS FIRE. USE A LOOKOUT AND CALL FOR FALLERS. - MONITOR VISIBILITY. WHEN VISABILITY IS BAD, IT COMPROMISES THE ABLITY OF YOUR LOOKOUTS TO SEE DANGER AND COMMUNICATE IT. WHEN VISABILITY BECOMES GOOD, YOUR FIRE IS ABOUT TO TAKE OFF! |  |