| FIRE BEHAVIOR FORECAST |  |
| --- | --- |
| FORECAST NUMBER 8 | TYPE OF FIRE: Timber |
| FIRE NAME BTU Lightning | OPERATIONAL PERIOD July 5, 2008 |
| DATE ISSUED 7/4/2008 | TIME ISSUED 1900 |
| UNIT: Cal Fire – Butte Unit | SIGNED David Kerr |
| INPUTS |  |
| WEATHER SUMMARY: Warming and with slight drying as high pressure builds in from the north … however the inversion returns to the fire area. Expect stronger down canyon winds with very poor RH recovery. This warming tend continues through Wednesday. See the Incident Weather Forecast for details. |  |
| FIRE BEHAVIOR |  |
| GENERAL: The inversion will build back over the fire area today lifting to near 6,000 feet in the afternoon. This will create smoky air over the Complex. The large size class fuels are very receptive to ignition and can be used as target fuels for burning, but also create spotting issues due to this receptiveness. Stronger down canyon winds will develop tonight and has the potential to effect the lowed on third of the slopes in the lower Feather River Canyon. Moderate intensity upslope runs are likely when the fire repositions from backing orientation. |  |
| SPECIFIC: Camp Fire – Firing along the north/slope running portion of the U-Line Road may need to be finished in the early hours of this shift. Conditions will be favorable until about 1000 hours when the upslope winds begin to increase. Firing the southern portion of the indirect line from Flea Mountain to Hwy 70 is most favorable once the up canyon winds begin to develop in the late morning. This normally occurs around 1100hrs. Firing the upper portions of this line could proceed earlier as this area will be out of the influence of the strong down canyon winds. Expect strong down canyon winds to develop in the Feather River tonight and will adversely impact holding and firing operations in the river corridor. Pit Fire - The fire continues walking through the heavy dead/down material of the 2000 Storrie fire. The Pit fire is spreading mostly through roll-out on the steep slopes and the effects of the down canyon flow that influences the drainage. As the fire repositions upslope runs should be expected. The upper elevations of the fire area are green and wet and will eventually check spread in that direction. The heavy snag component creates a fire environment that is more supportive of fire brand generation and therefore short range spotting. Belden Fire - The fire is boxed in between Chips Creek on the west, Indian Creek on the east, the Feather River on the south and fairly green fuels north of the fire. Between these boundaries the fire will condition to spread through a series of roll-out and upslope runs and lateral spread driven by the canyon winds. No significant activity is expected today, however the fire should back to the Hwy 70, Feather River corridor today. |  |
| AIR OPERATIONS: Lots of residual smoke from last night burning operation will be trapped under the building inversion. Visibility should be best at ridgetops early in the shift and degrade during the day. |  |
| SAFETY - Slope reversal is real and continues to be the greatest threat to ground crews. As the fire moves around on the landscape the potential of upslope runs increases as new fuelbeds come into alignment. - Care needs to be taken during all firing operations to control intensity. Building too much fire intensity by lighting too fast or too much fuel will cause torching and increase the potential of losing the burn. - Building nighttime down canyon winds in the Feather River Canyon may create issue with holding operations on the south end of the Camp Fire. Crew need to be briefed on the wind patterns. See the Incident Weather forecast for details. |  |