Mendocino National Forest

Aviation Safety & Operations Plan

Plastic Sphere Dispenser & Helitorch

FY 2007-2012

Prepared by: /s/ Thomas Caves                Date:  02/13/2007

Thomas B. Caves        	  Forest Fuels Officer

Reviewed by: /s/                                             Date:

David Sinclear 	              Forest Fire Management Officer

Reviewed by: /s/                                             Date:

Steve Decker      	             Forest Aviation officer

Approved by:_/s/                                       Date:

Thomas A Contreras          	 Forest Supervisor

# Contents

### Plastic Sphere Dispenser

1-  	Organization/Position Responsibilities

2- 	Helibase Briefing

3- 	Emergency Plan

4- 	Plastic Sphere Dispenser Operation Checklist

5-	Flight Procedures, firing procedures PSD

6-	Certification

#### Helitorch

7-	Helitorch Module

8-	Requirements, restrictions & approvals

9-	Operational Sequence

10-	Helibase Operational Procedures

11-	Mixing Safety gas & gel

12-	Helibase Safety

13-	Crash Rescue Plan

14-	Communications Plan

15-	Helibase Operations Checklist

16-	JHA

17-	Certification

Organization/Position Responsibilities

Plastic Sphere Dispenser Operations - Prescribed & Wildland Fire

1.  PILOT:

Pilot will follow lighting plan, under the direction of the Burn Boss/Ignition Specialist.  Pilot-in-command is responsible for all matters related to aircraft operations and safety, including installing Plastic Sphere Dispenser (PSD) in helicopter and helicopter load calculations.

2.  PSD OPERATIOR:

Serves as PSD Operator for the Burn Boss/Ignition Specialist.  Briefs pilot, identifies safety requirements at the operations briefing, and monitors overall operation.  May serve as Helibase/Helispot Manger.  The operator is responsible for the preparation, operation, maintenance, and care of the PSD.  The operator verifies for Burn Boss/Ignition Specialist that prescribed spacing of ignition is occurring and makes necessary adjustments.  Determines if malfunction occurs and acts accordingly.  If onboard fire occurs must determine if fire can be extinguished, or if unit must be jettisoned.  The operator will communicate with the pilot and ground personnel on all procedures associated with operation and or emergencies occurring during operation.

3.  PARKING TENDER:   (optional)

Directs pilot during landing and take-off by use of radio.  Equipped with a boom microphone and heat-set.  Insures that all personnel are clear of safety circle during take-off and landing.

4.  HELIBASE/HELISPOT SUPPORT (as needed – determined by Burn Boss and Helibase Manager.)

- Helibase/Helispot fire protection: At a minimum, one 2lb fire extinguisher and 5 gallons of water will be positioned at the helibase/helispot.

- Radio Operator: Will be at the helibase/helispot.  Will initiate radio communications with burn boss and dispatch.

Helibase Briefing

Plastic Sphere Dispenser - Prescribed & Wildland Fire

1- Attendance: 		Pilot

PSD Operator/Helibase/Helispot Manger

Parking Tender

Burn Boss/Ignition Specialist

2- Pilot has maps/photos:

Flight Hazard Map (displayed)

Discuss restricted areas

3- Define roles performed by each person/position

4- Standard communication terminology - demonstrate both hand and radio communications

5- Actions to be taken by each individual in event of mishap.  Display and demonstrate crash rescue kit contents and use.  Have pilot point out location of main battery switch and fuel supply shut-off.

6- Discuss ignition patterns and firing sequence

7- Location of Burn Boss/Ignition Specialist

8- Critique previous day’s operation (if applicable)

# Emergency Plan

Plastic Sphere Dispenser - Prescribed & Wildland Fire

I.  EMERGENCIES:

A.	Notify Division/Branch

1.  Type of emergency

2.  Number of people involved

II. AIRCRAFT ACCIDENT:

- Extrication

- Use of crash rescue kit

- Use care to prevent further injury to aircraft occupants when using these tools.

- Shut off power by moving main battery switch to off position.

- Shut off fuel supply.

- Suspect serious injuries, exercise extreme care when removing occupants.

- Fire Suppression

  - Utilize foam and/or dry chemical or halon fire Extinguishers.

  - Give priority to protecting personnel.

III. FIRE AT MIXING AREA

- Give priority to protecting personnel

- Use fire extinguishers and water

- Shut off machine

###### IV. RESCUE/FIRE SUPPRESSION CREW

- Helibase/helispot manager

- PSD Operator

- Parking Tender (if assigned)

IV. FIRST AID AND TRANSPORT TO MEDICAL FACILITY

- Contact ECC, Burn Boss and or District/Forest to arrange transportation of injured personnel to nearest medical facility.

- After extrication/rescue of personnel administer first aid as needed.

Plastic Sphere Dispenser - Checklist

HELIBASE SAFETY

- Qualified PSD Manager/Operator assigned

- Helibase/helispot meet established standards

- Organizational chart posted, assignments known

- Communications chart posted.  Frequency assignments known

- Helibase/helispot fire protection meets established standards

- Crash rescue/evacuation kits on the helibase/helispot

- Emergency Plan posted

- All personnel briefed, aerial ignition personnel briefed on in flight operations

- Separation of aircraft (if more than one used)

- Personal protective equipment meets established standards

AIRCRAFT/PILOTS (S)

- Check pilot and aircraft, Office of Aircraft Services (OAS) approval cards

- Check pilot and aircraft limitations

- Load calculations prepared and posted

- Check aircraft radios

- Remove all loose articles from aircraft

- Fire shelter on board aircraft for each person

- PPE for all persons and pilot

- Water bucket ordered with aircraft (optional)

###### PLASTIC SPHERE DISPENSER

- Installation correct with restraints in place

- Mechanical operation satisfactory

- Extinguisher (water reservoir) system filled and operational

- Glycol reservoir filled and tightly capped

- 20 second ignition delay achieved

- Intercom and aircraft-to-ground communications operable

- Pilot has been briefed and agrees that all is in order

- Sphere containers secured

- Knife available for emergency use

- Additional container of water available

- Tool kit/Premo Mark III manual on board aircraft (optional)

###### BURNING OPERATIONS

- All persons briefed and assignments known

- Maps/photos of project area used/posted

- Special weather considerations known and discussed

- Communications plan posted and frequency assignments known

- Emergency Operations plan known an discussed

- Personal protective equipment meets established standards

- Special safety considerations know and discussed

###### SUPPORT EQUIPMENT/PERSONNEL

- Adequate support equipment/personnel to complete mission

- Pump/engine operational checks

- Radios/communications operational checks

- Radios/communications operationally checks

- Support equipment /personnel propositioned before actual operations begin

- Adequate supply of plastics spheres and glycol to complete project

- PSD checklist complete

The following signatures certify that all of the above checklist items have been accomplished.

_________________________________    	_________________________

PSD Operator					Date

_________________________________	_________________________

Burn Boss					Date

##### Flight Procedures

Plastic Sphere Dispenser - Prescribed & Wildland Fire

Flight Procedures

- Firing - Plastic Sphere Dispenser

- Conduct firing as closely as possible to the plan described during the briefing.

- The Burn Boss or ignition Specialist, if assigned shall direct the helicopter on all lighting patterns.  Use of a higher vantage point or another helicopter or fixed-wing aircraft can be used to do this.

- The Burn Boss shall be kept informed of any changes that occur, and shall keep the pilot informed of crew activities and location.

- All burning operations shall be conducted in accordance with FAR part 137, restricted category agricultural aircraft operations.

- The maximum recommended speed should be 50 mph or 42 knots.  Slow aircraft speed to planned application speed while firing operation is in progress.

- Dispensing flight altitude should be 300 feet above the ground.

- Occupants of the helicopter shall be limited to the pilot, dispenser operator, and firing boss if essential to the mission.

- Make a dry run over burn area to ensure communications between all parties involved, no people are in the area and to identify boundaries of burn area.

- After a dry run over the planned burn area, pilot and operator should determine the firing pattern as instructed by the Burn Boss/Ignition Specialist.  Section 45.3 of the Helicopter Operations Handbook outlines various speeds and altitudes that may be used.

PSD Operation - Prestart the dispenser as follows:

- Turn hopper feed

- Turn on glycol pump

- Lift appropriate feed control handles to provide desired spacing of ignition points on the ground.

- Enter first flight line at operation speed and turn on drive switch.

- Prevention of ignition outside the burn area.  Follow these procedures:

- Operate the aircraft inside the burn area

- Close the chute feed handles

- Ensure that all plastic spheres in the chutes have been dispensed into the burn area

- Turn off all switches

- Aircraft may then leave burn area

- Continue to monitor the dispenser to ensure no injected spheres remain in the hopper.

- Follow malfunction procedures of hang up or other problems.

- Care should be taken to prevent contamination of streams and lakes.

- Equipment Malfunctions

- Occasionally, a sphere or fragment will jam on entry or otherwise prevent movement of a slipper block, causing the dispenser to stall.  Immediately turn off the motor to prevent damage.  Pull the manual assist wheel outward to disengage the motor and rotate forward and backward until machine is clear, then restart the motor.  Do not remove the hopper to warn operator.

- Interruption of operation caused by an obstruction, or power or motor failure will result in primed sphere remaining within the dispenser.

- notify pilot

- push down feed control feed handles

- turn off drive motor

- pull manual assist wheel outward and resume operations

- If fire starts, depress the red push button (emergency water) and hold 30 seconds.  If necessary, use canteen of additional water to extinguish fire by pouring down feeder chutes through hopper.  If problem persists, land.

- Remove hopper and feed chutes.

- Clear all chambers.  Dispose of all debris overboard.

- Check replace fuse if necessary

- If the fire proves uncontrollable, the dispenser and all items must be jettisoned, unless the emergency occurs over a developed area.  The pilot must be aware that a fire exists, and must direct that the dispenser be jettisoned.  To jettison, cut the restraining belt on the outboard side of buckle with the knife, grasp the dispenser by handle, and tip clear of skids.

Helitorch Use

Prescribed & Wildland Fire

I.	Requirements, Restrictions, Approvals

- The helicopter and Pilot must be approved for helitorch use and FAA approved for external load operations.

- Pilots and ground personnel accomplishing the mixing, loading, and application of the gelled fuel will be qualified to perform helitorch operations and understand personnel responsibilities outlined in the Interagency Aerial Ignition Guide.

- All personnel involved in the project will be thoroughly briefed in helicopter safety and the helitorch operation on the morning of the burn prior to flights of any kind.

- All elements of the helitorch module will be certified as qualified by the helicopter operator for helitorch operations.  The helitorch must be built or modified to current standards identified for aerial ignition devices.

- All helitorch/helicopter operations will be conducted in accordance with this plan and a Job Hazard Analysis for the prescribed burn.

- A discrete radio frequency is required for any helitorch operation.

- The helitorch fuel mixing operation will be done in accordance with this safety plan regarding handling, mixing, transportation, and storage of the fuel.

II.   Operational Sequence

- Training

- Assemble helitorch module personnel and equipment.

- Conduct training and review session on the helitorch, assuring that all necessary equipment and supplies are available and in proper working order for the day of the burn and that all personnel are thoroughly familiar with operational procedures and equipment maintenance.

- Day prior to burn (or early morning on the day of the burn)

- Deliver necessary equipment and supplies to project helibase.

- Set up helibase, i.e., landing area, parking area, mixing area, crash rescue, etc.

- Day of Burn

- Burn boss holds a briefing on the project explaining in detail, i.e., specific duties of each person, communications plan, JHA, flight paths and patterns, emergency procedures.  All flight hazards will be reviewed and discussed.

- Helitorch manager conducts operational and safety procedures briefing with all personnel concerning helicopter and torch operations and safety.

- Make an orientation flight with the pilot and helitorch manager/lighting boss.  The flight will include:

- Reconnaissance of burn site

- Flight patterns

- Ignition patterns

- Local hazards

- Water sources for dipping operations

- Personnel and equipment locations during the burn

- Location of project helispots and landing areas

- Review size, shape, number and location of burn areas

- Mixing of helitorch fuel and hookup of helitorch will be done and helitorch operation will be checked on the ground.

- Burn Boss will radio to helibase when ready for ignition to begin.  All radio communication will be rechecked.

- Helitorch operation will begin under close direction of the Burning Boss.

- When ignition is completed, helicopter will return to helibase and shut down at the discretion of the Burning Boss.

- Post operational safety briefing will be conducted by the Prescribed Fire Burn Boss and the Helitorch Manager.

III.  Helibase Operational Procedures

- Fuel truck and mix area will be outside of the safety circle - truck and tanks, etc. 50 feet or more from helicopter landing area.

- Position AFFF foam equipped engines away from fuel truck, mixing area, and clear of helicopter approach/departure path.  Engines must be equipped with foam approved for extinguishing petroleum fires.

- Helicopter will not take off or land unless landing area is clear of all personnel.

- The Parking Tender will supervise landings and takeoffs with the direction of the helicopter pilot.

- Unloading /loading crew will clear landing area perimeter immediately after completion of loading and will not enter landing area except when signaled to do so by the Pilot.

- The helitorch shall be positioned in front of the helicopter, for hookup and liftoff.

- If cables tangle or other problems occur requiring adjustment of the helitorch or its suspension system the helicopter Pilot shall be instructed to land or jettison the helitorch.  The helitorch shall not be flown over personnel on the ground at any time.

- All mix barrels being used will be grounded at all times except when being transported.

- Spare barrels will be stored away from the mixing area and outside the safety area.

- The foam (AFFF) engine assigned will be staffed and prepared to take suppression action during helicopter operations and mixing and loading of fuel.

- No one is allowed in the mixing or landing area except those authorized by the Helitorch manager.

- The helicopter is under the control of the Helitorch manager.  However, the Pilot has the final say and shall not be pressured in any way to do something that he/she is not comfortable with.

IV.  Mixing Safety Gasoline and Gel

- Helitorch Manager will appoint a Mixmaster who is in direct control of mixing operations.

- The Mixmaster sees that:

- Tanks are grounded at all times except when being transported.

- Gasoline-gel quantities are exact.

- Gasoline nozzle is completely shut off before removal from tank being filled.

- The gelling agent must be kept dry.

- Mix Crew - only the number of people necessary to conduct mixing and transporting functions will be in the mixing and helitorch area.

- General

- All members of the mixing-transporting job will wear safety clothing consisting of hardhat with chin strap, cotton coveralls, hearing protectors, safety goggles, organic vapor respirator, gloves, field boots at all times while engaged in the mixing-transporting operation.  The mixing crew due to the possibility of static electricity discharge will wear no nylon or synthetic fabric.

- The foam (AFFF) engine assigned to the helibase will be placed in a location that provides quick access to both the mixing area and the helicopter -helitorch area.  Pre-laid hose may be used where appropriate.

- The Helitorch manager will establish escape routes and all helibase personnel will know where to go and what to do in the event of an emergency.

- No smoking is permitted within the mixing/loading area.   Precautions must be taken to eliminate sources of ignition.

- In the event that an emergency occurs procedures outlined in the Crash Rescue Plan will be followed.

## IV.  Helibase Safety

- Helibase Manager is directly responsible for helibase safety.  This includes:

- All personnel clear of the landing area during all takeoffs and landings. At no time should there be any one underneath of the helicopter with the helitorch attached.

- Mix crew is in the helitorch area only as long as required for loading and unloading torch.

- Mixing operation is conducted safely.

- Assures proper communications are established according to the communications plan.

- Foam (AFFF) engine is properly positioned to provide emergency services to both mixing and helicopter functions.

- All helibase personnel are properly equipped with prescribed safety equipment.

- Hard-hats with chinstraps

- Hearing protection

- safety goggles

- Organic vapor respirator

- Cotton coveralls for the mixers

- Boots

- Gloves

CRASH RESCUE PLAN

Helitorch – Prescribed & Wildland Fire

In the event of an accident during the project the Helitorch Manager or Burn Boss will supervise and coordinate the crash rescue activities.  Specific crash rescue duties will be assigned to helibase personnel each morning before flights of any kind.  Crash rescue and first aid equipment will be located on or near the engine assigned to the helibase and the equipment location made known to all helibase personnel.

The local fire Department will be notified immediately and assistance requested.  All requests for assistance will be coordinated through the Incident Command Team or Prescribed Fire Burn Boss and the ECC.

COMMUNICATIONS PLAN

Helitorch – Prescribed & Wildland Fire

There will be a separate discrete frequency used for communication between the Pilot, Burn Boss, Helibase Manager, and Helitorch Manager during helitorch operations.

Flight following will be done on-site during helitorch operations

All communications will be checked prior to start of operation and rechecked just prior to start of the operation

HELITORCH OPERATIONS CHECKLIST

This Helitorch Operations Checklist shall be completed prior to each day’s operation.

- Organization

- Helitack organization chart has been prepared and posted showing responsibilities for functions named.

- All positions are filled by qualified

Personnel

B.  Helibase Operations

- Helibase Manager

- Has established separate radio

Frequencies as designated on Comm. Plan

- Communications tested and

Operational

- Briefings to include all required

Helitack personnel, key firing

Personnel and helitorch Pilot

- Overhead responsibilities and authorities

- Flight routes

- Area Flight Hazards

- Radio Frequencies

- Personnel assignments

- Emergency procedures

NOTE: All personnel will be briefed on the adherence of alumagel and the hazards from coming into contact with it and gasoline

C.  Mixing Area

- Separate from Helibase activities

- Traffic control

- Helitorch fuel supply available and properly located and grounded

- Fire Suppression equipment available

- Personnel assignments

- Emergency procedures reviewed

- Landing Area

- Located where safe approach and departure paths exist, and separate from other helibase activities take place

- Free from flight hazards

- Traffic control of ground vehicles, personnel, and aircraft

- Dust abatement

- Helicopter fuel truck security - parking area and driving route designated, located away from flight routes, landing areas and personnel.  Static grounding measures required

- Helitorch loading procedures briefed

- Fire suppression equipment available

- Fire suppression person available

- Orientation flight completed

- Go/no-go check list completed

CERTIFICATION - All items of the Helitorch operations Checklist have been accomplished.

_______________________________		____________________

Helitorch Manager					Date

_______________________________		____________________

Prescribed Fire Burn Boss				Date

This Plan was developed within the framework of the Federal Interagency Aerial Ignition Guide, September 1998

Job Hazard Analysis

Plastic Sphere Dispenser - Prescribed & Wildland Fire

- Work Project/Activity:  Plastic Sphere Dispenser firing

- Location:  various projects

- Unit:  Mendocino National Forest

- Name of Analyst:  Thomas Caves

- Job Title:  Fire Management Specialist

- Date Prepared: May 1998, Reviewed Feb 2007.

Tasks/Procedures:

- Qualifications

- Responsibilities

- Bench Testing

- Personal protective equipment

- Malfunctions

- Ignition

- Flight following

Hazards:

- Unqualified personnel

- Unknown responsibilities

- Injuries

- Possible fire in machine, machine not operating properly

- Firing pattern/fire behavior

- Loss of communication

Abatement Actions:

- PSD operator shall be certified annually.  Pilot and helicopter will be certified annually for aerial ignition.  Pilot will have two hours fire behavior and trained in use of the fire shelter.

- Prior to each project, operator will review appropriate portions of IHOG and Interagency Aerial Ignition Guide.  The project briefing will cover responsibilities and emergency procedures.

- Bench testing will be done prior to mounting the machine in the helicopter and will be done at a safe distance from the helicopter.

- Operator and pilot shall have a fire shelter and wear flight helmet, nomex flight suit, nomex gloves and leather boots.  The Burn Boss will be required to have the same PPE if riding in the helicopter.

- Malfunctions will be addressed in the project briefing.  Operator will immediately notify pilot of malfunction and take appropriate actions to correct it.   If malfunction cannot be corrected in the air, the helicopter will set down.  If a fire occurs in the machine will be jettisoned.

- Firing pattern will be determined by the Burn Boss to ensure objectives are met safely.

- Flight following will be done from the helibase during the project.  Upon completion of the project, flight following will be turned over to the appropriate unit/dispatch.

Reference: Interagency Aerial Ignition Guide 9/99 for Helitorch JHA