| Incident Name: Siskiyou Complex, Blue 2, Ukonom | IR Interpreter(s): Jim Grace | Local Dispatch Phone: Yreka 530-842-3380 | Interpreted Size: 121,386 Growth last period: 832 acres |
| --- | --- | --- | --- |
| Flight Time: 2109 MDT Flight Date: 08-10-08 | Interpreter(s) location: Redding, Ca Interpreter(s) Phone: 530-226-2751 | Regional Coordinator: Guy Lewis Regional Coord. Phone: 530-598-1951 | National Coordinator: Darryle VanBurren National Coord. Phone: 208-387-5381 |
| Ordered By: Erik Stemmerman | Order Number: Siskiyou A#: 167 Blue 2 A-169 Ukonom A-168 | Aircraft/Scanner System: 149Z/Phoenix | Pilots/Techs: B.Mank/ Woody Smith |
| IRIN Comments on imagery: |  | Weather at time of flight: Clear | Flight Objective: fire perimeter mapping and heat detections |
| Date and Time Imagery Received by Interpreter: 08/10/2008 2300 hrs |  | Type of media for final product: .pdf, and shapefiles, and IR LOG Electronic file sent to: ftp://nifc.gov/Incident_Specific_Data/Calif_N/2008_North_Ops/CA-KNF-xxx_Siskiyou_Complex/GIS/IR |  |
| Date and Time Imagery Delivered to Incident: 08/11/2008 0400 |  |  |  |
| Comments /notes on tonight’s mission and this interpretation: The isolated heat reported last night Northeast end of Ukonom, South of Monment Lake, has grown in size and intensity Tonight. Long -123 21 Lat 41 31.1. Blue - The Northeast end of Blue 2 has areas of intense heat and areas of perimeter growth down into the Blue Creek drainage. The Southwest end of the fire has scattered heat at the Southern tip and some small areas of intense heat in the Crescent City Fork Blue Creek. Siskiyou - Intense heat was detected North of Little Medicine Mtn and West of the Mtn along the Western fire perimeter. Intense heat was also detected in the Medicine Creek drainage along the western perimeter. Isolated heat and a small area of Intense heat were detected in the Flint Valley area. Isolated and intense heat was detected in the upper East Fork Blue Creek drainage. Also, Intense and isolated heat was detected on the West perimeter of the fire between Wildcat and East Fork Creeks. Small areas of intense heat and some isolated heat was detected on the Southwest finger of the fire that runs along Lonesome Ridge out to Nickowitz Peak. The Western most part of the fire west of Nickowitz peak, intense heat from a burnout operation was detected on the top of the ridge above Blue Creek. Ukonom - The North end of the fire and the Eastern most part of the fire continue to hold the majority of the heat. On the North, intense heat was detected South of Haypress Meadows and along the east perimeter Southeast of Sandy Ridge. The isolated heat south of Monument increased in size and heat intensity. Another area of intense heat was located at the top of the ridge South of Snowslide Gulch. A large area of scattered isolated heat was detected North and West of the confluence of Halfmoon and Bridge Creeks. Isolated heat sources were detected North of Black Mtn along an interior island. On the Eastern most portion of the fire. Isolated heat and intense heat was detected around Chimney Rock and North and East of Chimney Rock. Another area of intense heat was detected West of Crapo Mtn along what the USGS Quad calls the Garden Gulch Trail, at the top of the ridge above the trail. |  |  |  |