| Incident Name: Siskiyou Complex, Blue 2, Ukonom | IR Interpreter(s): Jim Grace | Local Dispatch Phone: Yreka 530-842-3380 | Interpreted Size: 123,104 Growth last period: 599 acres |
| --- | --- | --- | --- |
| Flight Time: 2007 MDT Flight Date: 08-12-08 | Interpreter(s) location: Redding, Ca Interpreter(s) Phone: 530-226-2751 | Regional Coordinator: Guy Lewis Regional Coord. Phone: 530-598-1951 | National Coordinator: Darryle VanBurren National Coord. Phone: 208-387-5381 |
| Ordered By: Erik Stemmerman | Order Number: Siskiyou A#: 173 Blue 2 A-175 Ukonom A-174 | Aircraft/Scanner System: 149Z/Phoenix | Pilots/Techs: B.Mank/ Woody Smith |
| IRIN Comments on imagery: |  | Weather at time of flight: Clear | Flight Objective: fire perimeter mapping and heat detections |
| Date and Time Imagery Received by Interpreter: 08/12/2008 2200 hrs |  | Type of media for final product: .pdf, and shapefiles, and IR LOG Electronic file sent to: ftp://nifc.gov/Incident_Specific_Data/Calif_N/2008_North_Ops/CA-KNF-xxx_Siskiyou_Complex/GIS/IR |  |
| Date and Time Imagery Delivered to Incident: 08/13/2008 0400 |  |  |  |
| Comments /notes on tonight’s mission and this interpretation: Blue 2 - Heat continues to be detected in the Northeast and Southwest corners of the fire. The Northeast finger of the fire continues to have some pockets of intense heat as the perimeter has moved downhill into the Blue Creek drainage. There was some isolated heat and a small area of intense heat detected Southeast of Doctor Rock and South of a sherlter as shown on the USGS topo quad. The Southwest corner of the fire has isolated and some small pockets of intense heat in the Crescent City Fork Blue Creek and some intense heat in the Southern most portion of the fire. Siskiyou - an isolated heat source was detected outside of the main heat perimeter on the Norhtwaest end of the fire on Dillon Divide Southwest of the confluence of Lick and North Fork Dillon Creeks. This is the same area that the SITL talked to me about 2 days ago as heat showing outside of the control line. It was discovered that the control line was in an approximate location. This isolated heat may or may not be inside the actual control line. It will need to be field verified to make sure. Intense heat was deteced Northeast of Little Medicine Mtn. in the Dillon Creek drainage. Intense and isolated heat sources where detected along the perimeter West of Little Medicine Mtn. West and South along the boundary to just South of where the perimeter crosses Medicine Creek. The Flint Valley area had a few isolated heat sources. The upper end of East Fork Blue Creek, Southwest of Flint Valley had some pockets of intense heat and isolated heat. The area Southeast of the confluence of East Fork Blue Creek and Wildcat Creek had some pockets of intense heat and areas of isolated heat sources. The Southwest finger of the fire that runs out Lonesome Ridge out to and past Nickowitz Peak, showed isolated heat in the East Fork Blue Creek drainage as well as some intense heat North of Nickowitz Peak and West of the peak along the heat perimeter. Ukonom - The finger of fire, North end of the fire, had numerous isolated heat sources as well as a few pockets of intense heat. South of Haypress Meadows and North of Camp 4, there were a few pockets of intense heat and a few isolated heat sources. The heat perimeter South and west of Monument Lake appears to have crossed West of the handline that is constructed in this area. |  |  |  |