| - Incident Name: | - Yuba River Complex | - Analysis Tier Level: | - Tier I – Primary Assets Only |
| --- | --- | --- | --- |
| - State(s): | - California | - FSPro Projected From: - RAVAR Requested: | - 23 June 2008 - 23 June 2008 |
| - Report Date: | - 23 June 2008 |  | - 7-day projection |
| - RAVAR Analyst: | - Kevin Hyde (METI) RMRS - Missoula – 406.329.2137 - kdhyde@fs.fed.us | - FSPro Analyst: | - Larry Hood R5 - lhood@fs.fed.us - 530 598-1226 |
| - RAVAR Code: | - A_T1_0870623 kh |  |  |

NOTE: This report accompanies a RAVAR analysis map.

- All RAVAR products are intended for STRATEGIC use only. It is always advised that values analyzed and displayed on RAVAR maps be verified by local knowledge.

- FSPro Basis for Analysis:

- FSPro Analysis – 7 day starting 23 June 2008; Projected spread assumes NO further suppression

- NOTE: The < 1% spread probability zone is displayed on the map (pale pink zone) but assets within are not summarized. This zone represents “the rare event” – confidence in this prediction is undetermined.

- Tier I Analysis – Primary Assets Only

- General Assessment:

- The four major fires of this complex mapped and described in this report are burning on the Tahoe National Forest in Northern California. Based on the best understanding of fire names:

  - Fall Fire is located 1 mile E of Washington and 4 miles W of the Clear (possibly Scotchman?) Fire

  - Clear Fire is located 3 miles SW of Bowman Lake

  - Celina Fire is located 3 miles NW of Graniteville and 8 miles N of the Clear Fire

  - 25 Fire is located immediately NE of Cal-Ida

- Assuming no further suppression, the 7-day fire spread projection indicates 20-40 probability that the Fall and Clear Fires will merge; 1-5% probability that the Celina and Fall/Clear Fires will merge; the 25 Fire will spread predominately to the NE

- Communities threatened: Multiple remote communities – many “Historical” are threatened

  - Washington to W of Fall Fire within 20% probability zone

  - Gaston between Fall/Clear Fires within 5-20% spread zone

  - Graniteville between Fall/Clear/Celina Fires within the “rare event” zone

  - American Hill (historical) E of Celina Fire at transition into 60-80% spread zone

  - Brandy City NW of 25 Fire within 4-60% spread zone

  - Mount Pleasant NNE of Cal-Ida at edge of “rare event”zone

  - NOTE: Communities of Bowman, Fuller Lake, and Rucker are listed on the 209 of this date as threatened but could not be identified with maps and data available

  - Note relative density of structures threatened (high or low) and location relative to FS Pro probability zones

- Structures threatened primarily in Sierra County, few in Nevada County

- *** Report continues on next page ***

- Major Infrastructure:

  - The Hwy. 49 corridor is directly threatened by the Celina Fire; the I-80 corridor is S of the Fall/Clear Fires, outside spread probability zones

  - Oil and Gas Pipelines – located along I-80 corridor outside spread zones

  - Powerlines – Multiple powerlines identified within all potential spread zones

- Water Resources:

  - Water Supply Infrastructure –

    - Multiple water supply and hydropower dams lie E of the Fall/Clear Fires within or proximate to the lowest spread probability zones. All features are upstream of the fires and therefore are not expected to be threatened by possible post-fire erosion

    - The New Bullards Bar and the Scotts Flat Reservoirs are located 10 miles downstream to the SW

  - Aqueducts and pipelines throughout spread zones

- Other resources threatened:

  - Multiple Other Landmarks are threatened within all spread zones

- Note: Multiple mines are located throughout the area and may pose safety hazards to firefighters

- *** Report continues on next page ***

- Values assessed in this Report:

- NOTE: Analysis of assets within the 80% Spread Probability Zone includes values-at-risk that may be within the active fire perimeter; any assets within the active perimeter are not independently analyzed.

- Primary land jurisdictions (Table 1)

- Subtotals of USFS jurisdictions within designated roadless or wilderness areas (Table 2)

- Total structure estimate and area by county (Table 3)

- Building cluster points per GIS parcel layers of Nevada Count (Table 4)

- Building structure points extracted from air photo interpretation by the USGS for Sierra County (Table 5)

- Aqueducts and water pipelines (Table 6)

- Powerlines (Table 7)

- Other Landmarks (Table 8)

- Jurisdictions within FSPro Spread Zones:

Table 2

| Jurisdiction | USFS - Total |  | State |  | Private |  |
| --- | --- | --- | --- | --- | --- | --- |
| FSPro Zone | In Zone | Cum. | In Zone | Cum. | In Zone | Cum. |
| > 80% | 3,820 | 3,820 | - | - | 1,956 | 1,956 |
| 60 - 80 % | 2,303 | 6,122 | - | - | 1,044 | 3,000 |
| 40 - 60 % | 3,910 | 10,033 | 6 | 6 | 2,133 | 5,133 |
| 20 - 40 % | 7,432 | 17,464 | 47 | 53 | 4,151 | 9,284 |
| 5 - 20 %* | 15,245 | 32,709 | 25 | 78 | 6,058 | 15,342 |
| 1 - 5 %* | 34,586 | 67,294 | 36 | 114 | 16,609 | 31,951 |

- USFS Designated Roadless and Wilderness Areas: These acres are subtotals of USFS-Total jurisdiction acres

Table 3

| Jurisdiction | USFS - IRA |  |
| --- | --- | --- |
| FSPro Zone | In Zone | Cum. |
| > 80% | - | - |
| 60 - 80 % | - | - |
| 40 - 60 % | 68 | 68 |
| 20 - 40 % | 240 | 308 |
| 5 - 20 % | 882 | 1,190 |
| 1 - 5 % | 2,728 | 3,918 |

- *** Report continues on next page ***

- Estimates of Structure Values at Risk: Estimate is derived from analysis of Nevada County GIS data, count represents building clusters*; and structure points** derived from air photos for Sierra County.

- Note: Recreation residences and special use structures data are not available from USFS and are not identified or analyzed in this report for Nevada County; structures in USFS jurisdictions of Sierra County may be captured by USGS structure points.

Table 4- Combined County Total – Nevada and Sierra Counties

|  | Acres Threatened |  | COMBINED COUNTY TOTALS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fire Spread Probability Zone | Acres by Zone | Cumulative Acres | Count by Zone | Cumulative Count | Value by Zone | Cumulative Value |
| > 80% | 5,776 | 5,776 | 14 | 14 | $3,652,151 | $3,652,151 |
| 60 - 80 % | 3,347 | 9,122 | 2 | 16 | $396,538 | $4,048,689 |
| 40 - 60 % | 6,049 | 15,172 | 3 | 19 | $594,807 | $4,643,496 |
| 20 - 40 % | 11,629 | 26,801 | 11 | 30 | $2,180,959 | $6,824,455 |
| 5 - 20 % | 21,327 | 48,128 | 8 | 38 | $1,586,152 | $8,410,607 |
| 1 - 5 % | 47,251 | 95,379 | 26 | 64 | $5,154,994 | $13,565,601 |

- * Building Clusters represent the center of parcels where county assessor records indicate taxable improvements are present. One or more structures and/or other improvements may exist proximate to these point locations. Accuracy of cluster points decreases with large parcel areas.   Valuation of building clusters is based on inflation adjusted average home value of $373,546 for Nevada County per U.S. Census data.

- ** Structure points are derived by the USGS through supervised analysis of high resolution aerial photography. Structures within dense canopy may not be mapped and outbuildings not distinguishable from residential structures may be assigned full structure value. Valuation of structure points is based on inflation adjusted average home value of $198,269 for Sierra County.

- Commercial assets may be undervalued.

Table 5- Sierra County

|  | Acres Threatened |  | Structure Value Sierra County |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fire Spread Probability Zone | Acres by Zone | Cumulative Acres | Count by Zone | Cumulative Count | Value by Zone | Cumulative Value |
| > 80% | 1,523 | 1,523 | 9 | 9 | $1,784,421 | $1,784,421 |
| 60 - 80 % | 1,172 | 2,694 | 2 | 11 | $396,538 | $2,180,959 |
| 40 - 60 % | 2,456 | 5,150 | 3 | 14 | $594,807 | $2,775,766 |
| 20 - 40 % | 5,343 | 10,493 | 11 | 25 | $2,180,959 | $4,956,725 |
| 5 - 20 % | 12,220 | 22,713 | 8 | 33 | $1,586,152 | $6,542,877 |
| 1 - 5 % | 22,412 | 45,125 | 26 | 59 | $5,154,994 | $11,697,871 |

- **** Report continues on next page ****

Table 6- Nevada County

|  | Acres Threatened |  | Structure Value Nevada County |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fire Spread Probability Zone | Acres by Zone | Cumulative Acres | Count by Zone | Cumulative Count | Value by Zone | Cumulative Value |
| > 80% | 4,253 | 4,253 | 5 | 5 | $1,867,730 | $1,867,730 |
| 60 - 80 % | 2,175 | 6,428 | 0 | 5 | $0 | $1,867,730 |
| 40 - 60 % | 3,593 | 10,021 | 0 | 5 | $0 | $1,867,730 |
| 20 - 40 % | 6,287 | 16,308 | 0 | 5 | $0 | $1,867,730 |
| 5 - 20 % | 9,107 | 25,415 | 0 | 5 | $0 | $1,867,730 |
| 1 - 5 % | 24,840 | 50,254 | 0 | 5 | $0 | $1,867,730 |

- NOTE: 3954 acres are threatened in Yuba County at the 1 – 5 % probability level. No structures are identified in this area.

Table 7

| Aqueducts and Pipelines (mi) |  |  |
| --- | --- | --- |
| FSPro Zone | In Zone | Cum. |
| > 80% | 3 | 3 |
| 60 - 80 % | 1 | 4 |
| 40 - 60 % | 2 | 6 |
| 20 - 40 % | 1 | 7 |
| 5 - 20 % | 3 | 10 |
| 1 - 5 % | 3 | 14 |

- **** Report continues on next page ****

- Power Transmission Lines (mi):

Table 8

| Power Transmission Lines (mi) |  |  |
| --- | --- | --- |
| FSPro Zone | In Zone | Cum. |
| > 80% | 3 | 3 |
| 60 - 80 % | 1 | 4 |
| 40 - 60 % | 1 | 5 |
| 20 - 40 % | 2 | 7 |
| 5 - 20 % | 5 | 12 |
| 1 - 5 % | 7 | 19 |

- Other Landmarks:

- Table 8

| OTHER LANDMARKS | FSPro Zone |  | OTHER LANDMARKS | FSPro Zone |
| --- | --- | --- | --- | --- |
| Camp Nineteen | > 80 % |  | Camp - FS | 1 - 5 % |
| American Hill Cabin | 60 - 80 % |  | Camp Lundeen | 1 - 5 % |
| Picnic Area - FS | 40 - 60 % |  | Carr-Feeley Campground | 1 - 5 % |
| Lower Carton Campground | 40 - 60 % |  | Convict Flat Picnic Area | 1 - 5 % |
| Upper Carlton Campground | 40 - 60 % |  | Dolan Mine | 1 - 5 % |
| Golden Quartz Picnic Area | 20 - 40 % |  | Fuller Lake Campground | 1 - 5 % |
| Black Camp | 5 - 20 % |  | Galloway Ranch (historical) | 1 - 5 % |
| Bunker Hill Mine | 5 - 20 % |  | Lindsey Campground | 1 - 5 % |
| Camp - FS | 5 - 20 % |  | Old Condon Mill | 1 - 5 % |
| Cornish House | 5 - 20 % |  | Pliocene Mine | 1 - 5 % |
| Cornish House Campsite | 5 - 20 % |  | Quinn Ranch | 1 - 5 % |
| Fiddle Creek Camp Ground | 5 - 20 % |  | Robbins Ranch | 1 - 5 % |
| Halls Ranch (historical) | 5 - 20 % |  | Rose Ranch | 1 - 5 % |
| Halls Ranch Guard Station | 5 - 20 % |  | Trailhead | 1 - 5 % |
| Pineaire Campground | 5 - 20 % |  |  |  |
| River Rest Campground | 5 - 20 % |  |  |  |
| Shenanigan Campground | 5 - 20 % |  |  |  |

- END OF REPORT