| ## FIRE BEHAVIOR FORECAST |  |
| --- | --- |
| # FORECAST NUMBER: | TYPE OF FIRE: Wildland |
| FIRE NAME: Lockheed | OPERATIONAL PERIOD: 0700-0700 |
| DATE ISSUED: August 24-28, 2009 | TIME ISSUED: 0700 |
| UNIT: Santa Cruz (CZU) | SIGNED: Typed/printed: Don Boursier, FBTS |
| INPUTS |  |
| # WEATHER SUMMARY: # Morning low clouds and fog followed by partly cloudy conditions. SW winds should surface by afternoon. Daytime: Max Temps 60’s-70’s, Winds SW less than 10mph. Nighttime: Min Temps 50’s. |  |
| OUTPUTS # FIRE BEHAVIOR |  |
| GENERAL: As afternoon heating and drying occurs, the fire activity associated with 1 and 10 hour fuels could increase from mid to late afternoon. Fire activity will be primarily fuels/slope driven particularly in deep draws. Fuel Models 1, 2, 4, and 10 are present and mixed throughout the incident geographical area. Flame lengths from surface fuels may be up to 6 feet with rates of spread up to 10 chains per hour on south slopes in full alignment. Backing fire flame lengths are generally under 2 feet. A minimal potential for spotting will exist. The POI is 20% for all areas provided weather conditions remain consistent without any extremes. Spotting distances should not exceed ¼ mile. The greatest concern for spotting exists anytime torching trees are observed. Moderate night time RH recovery is expected on upper slopes and excellent on lower slopes. |  |
| SPECIFIC: Unburned fuel is located throughout the incident area but within the fire perimeter. Roll-out of hot material across established lines is still possible. |  |
| AIR OPERATIONS: Marine layer in the morning, then gradual clearing. |  |
| SAFETY Initial rapid increase in up slope/canyon winds upon inversion breakup may quickly intensify a creeping fires’ activity. Because the fire has now been present in timber fuels for a considerable time, watch for falling snags, ash pots/stump holes and underground root structure burn out. |  |