Potholes Trail Survey

Ojai Ranger District

October 31, 2007

Heidi Anderson

Approximately eighty percent of the six-mile Potholes Trail was burned over during the Ranch Fire.  The survey focused on the five mile section from the lower trailhead to the Potholes.

For preseason trail work, it is recommended that rolling dips are installed on the first two miles of trail and the two mile section that drops into the Potholes.  The second section will have more concerns with cross channels and significant rutting due to the slope of the hillside and steepness of the trail.

The dozer line mentioned are from the Piru Fire of 2003 and the Day Fire of 2006.  They seem to have little impact on the fires progress due to high winds.

Trail route was marked with green fluorescent flagging.

| Mileage | Elevation | Description |
| --- | --- | --- |
| 0.0 | 1,120’ | Lower Pothole Trailhead |
| 0.0 - .2 | 1,120’ – 1,220’ | Trail follows dozer line |
| .2 – 2.1 | 1,220’ – 2,660’ | The trail breaks off the north side of the dozer line for a short stretch; the remainder of the time is on the south side of the dozer line; rolling dips suggested to keep the water from rutting the trail when it rains. No rock is available for any type of waterbar structure. |
| 2.1 – 3.1 | 2,660’ – 3,200’ | Route follows the dozer line. No pre-work suggested |
| 3.1 – 5.3 | 3,200’ – 2,130’ | Trail follows the ridgeline for the first 1.5 miles and then switchbacks into the Pothole area. The primary concern is the amount of erosion on the switchback in to the Potholes. Rolling dips and waterbars are recommended. |