| ### FIRE BEHAVIOR FORECAST |  |  |
| --- | --- | --- |
| Fire: Santiago Fire | Unit: Cleveland NF, Orange Co | Prediction for 24 hr Shift |
| Forecast #13 Issued: 2000 11/5/07 | Signed: Dan Ardoin FBAN | # Shift Date: Tuesday 11/6/2007 |

| ## WEATHER SUMMARY: Conditions persist another day. Inversion lifting at 1000 hrs. |  |
| --- | --- |
| ## Max Temps: 83 ## Min RH : 18% | ## 20 Foot Winds ## Slope Winds: Up valley 6 -9 G 15 ## Ridge Winds: SE 7 to 11 G 15 |

### GENERAL FIRE BEHAVIOR Topography: Steep slopes. Valley and major canyons oriented East/West, elevation 1,000 ft rising to 5,000 ft at the top of the fire. Narrow V-Shaped canyons, box canyons, chutes, sharp ridges and saddles. Fuels: Grass and light to medium/heavy brush--extremely dry, live fuel moistures at critical levels; 30% to 50% live fuel moistures at lower elevations. Last winter was the driest ever recorded in this area. No significant rainfall for last 18 months. Extreme drought conditions exist and will persist. Fire does not need triggers, such as RH and wind to move… fire can be fuels and terrain driven and very active on any aspect. Lower Elevation ROS over 2 miles/hr with 20-40 ft Flame Lengths. POI 80% Higher Elevation ROS over 1 mile/hr with 20-25 ft FL. POI 50%

SPECIFIC FIRE BEHAVIOR Division QQ: Another day of drying for the fuels on the north aspects (live fuel moistures will be dropping). Slope winds cross the line. Short runs through fuel beds dried by backing surface fire are possible. These runs can bump and cross the line. Upslope winds from 1100-2000 hrs. All other divisions: Winds are in your favor. Burnout of fuels nearly complete; may be some sleepers especially in Div A (Pine Canyon) Downhill spread from rollout a possibility.

Air Operations: Visibility poor under the inversion until 1000 Sunrise: 0613 Sunset: 1655

SAFETY: Lookouts must be attentive at all times to observe changes in fire behavior, especially when there is reburn potential.