MNICS IMT GISS

STANDARD OPERATING GUIDELINES

Table of Contents

| Topic | Chapter | Page |
| --- | --- | --- |
| ArcMap-FIMT Tools | Chapter 1 | 2 |
| DNRGarmin | Chapter 2 | 7 |
| Google Earth | Chapter 3 | 9 |
| www.mnics.org | Chapter 4 | 10 |
| ftp.nifc.gov | Chapter 5 | 12 |
| www.inciweb.org | Chapter 6 | 14 |

ARCMAP, FIMT, and other EXTENSIONS

BACKGROUND

FIMT Tools have not been reprogrammed for ArcMap10, so be sure to bring a laptop with ArcMap 9.3.1 and all patches to an incident.

Load the following before an incident:

- FIMT extension

- US National Grid extension

- Turn on the Forestry symbology in ArcMap

OVERALL RECOMMENDATIONS:

It is recommended to do the following after each day, each planning cycle, or whenever necessary to protect data:

DIFFERENT VERSIONS OF FIMT:

Use the latest version of FIMT as they will have the most features. If a GISS cannot use the latest version of FIMT because they have an older version of ArcGIS, always transfer that GISSer’s work to another computer to fold it into the master Geodatabase.

SAVE, COMPACT, BACKUP:

- Save often and exit the software every 1-2 hours.

- Compact the Geodatabase often.  The file remembers all additions, modifications, and deletions, so it quickly increases in size.

  - In ArcCatalog, right click on the Geodatabase and choose “Compact Database”

- Always archive a backup copy of the Geodatabase and map documents.  With so much editing and work going on, they may be corrupted.  Developing the habit of regular backups will save time, effort, and embarrassment.

FIMT TOOLBAR

The FIMT tools are grouped into hierarchical categories on a single toolbar. They are laid out on the toolbar from left to right and are as follows:

- Incident information and Geodatabase

- Utilities dealing with metadata, zymology, labeling, etc., and working with selected features

- Firepolygon and Perimeter feature class tools

- Fireline feature class tools

- Firepoint feature class tools

FIMT works best in a left to right fashion. Be sure the incident information is up to date and the correct Incident Geodatabase is being edited (if more than one is loaded).  Continue sequentially through the groups and the application to ensure best performance.

- Adding Reference Data

Never load reference data into the map document prior to the creation and loading of an Incident Geodatabase.  It will cause map document problems.

- Manage by Polygon

When a fire perimeter has changed substantially, it is usually easier to delete the fire perimeter from the current Incident feature class (after being sure the feature has been copied to the history feature dataset).  Then either create a new Firepolygon or Fireline from scratch or import line or polygon data to form the new perimeter.

Note: Never try to update more than 50% of a firepolygon perimeter in one process.  Results are dramatic.

- Updating Fire Polygon with Line Data

If the user needs to bring in lines to update the polygon, the following is an easy way to update a polygon from a line:

- Just load the line theme into the ArcMap document.

- Select the polygon to edit, be sure the task is set to ”Reshape Feature” and target is set to ”Fire Polygon.”

- Using the Selection tools set the selectable layer to “Fire Polygon.”

- Activate the “Sketch Tool” (pencil).

- Place the cursor over the line from the new theme and right mouse click, choose “Replace Sketch.”

- If the line is going the same direction (e.g. right to left) as needed to update the polygon, then there will be a green point at the beginning of the line and a red point at the end of the line.  Complete line and then finish sketch by double clicking inside of the polygon (pressing the F2 key, or right mouse click, and choose Finish Sketch).

- If the line is going the opposite direction then needed, hover over the line, right click, and choose “Flip.”  This will change the direction of the line.  Now complete the polygon reshape.

- If there were any Assignment Breaks, they will be proportionally moved to the new perimeter.

- Finally, “Split Assignment Lines” in the correct locations.

- Assignment Breaks

Start at one location and work either clockwise or counter clockwise.  There is less changing of the attributes and the program does not become confused.

- More than One Incident Geodatabase Loaded

Be sure to use the “Switch Incident’ tool to ensure the correct Incident is being edited.

- History Feature Dataset

Turn the display of the History Feature dataset and the Selection of all Hist_ feature datasets off.  Since it is a Geodatabase and all data within the same workspace is editable at the same time, this will ensure no moving, splitting, or deleting of any information is inadvertently taking place.

- More than One Person Editing

Personal Geodatabases create a lock file and will not let more than one application (or person) work with the file at one time.  In such a situation, there are several alternatives.

  - Manage by Feature Class – export a Feature Class to a shapefile.  Allow one person to edit the shapefile.  User places a copy of the Incident.mdb in the background of the map document.  Add new fire lines to the shapefile.  When complete, the master editor for the incident can delete any appropriate fire lines and import the shapefile lines into the FC.  All of the attributes will also be imported.  The same is true for polygons and points.

  - Edit a copy of the Incident.mdb.  Begin by copying Incident data to the History feature dataset.  In the copy Incident Geodatabase, remove all features from the feature classes being edited, and add new features.  The master editor can then load the updates through the same method as above or by using the load tool within ArcCatalog.

  - Although the following is the least desired method, create a generic shapefile, add the features into the file, and follow the steps used in option “a”.  Because no attribute data is imported, the master editor will be required to go through each feature to correct the information.

  - The application will only work on a personal Geodatabase.  It will not work with SDE.

  - In the event a lot of data needs to be loaded, it may be easier to load (append) through ArcCatalog.  Right click on the Feature Class to be updated and choose the “Load Data” option.  Work through the wizard format.  Be sure the mapping of columns to the correct FIMT column name is correct.

- Auto Measures

The auto measure tool calculates the length for lines and the area for polygons by converting the features into an Albers Equal Area Projection, centering the feature in the spatial extent before the calculation is created.  This produces the smallest possible error in the calculation.  The standard area and length fields are present and are calculated using the projection defined for the Geodatabase.

- Firelines

Most users will convert the Perimeter Sector layer lines to Firelines.  This is a great place to start since the unit assignments will already be on the lines and they will be broken at the assignment breaks.  Split any lines needed to be assigned different Fire Line types.  Bring in any additional Firelines.

- Firepoints

Add all Fire Points into the Incident. The points will usually come from many different sources. Once complete, remember to use the “Populate Fire Point Coordinate Table” tool.  This will provide any map maker the ability to load the table on the map.

- Firepoint Coordinate Table

After all firepoints have been added, populate the FirePointCoordTable to assist in listing coordinate values for principle locations on the map.

PRODUCT WORKFLOW

- Create Transportation Map

  - The first information a GISS usually receives is the location description of a potential fire. From this information, a transportation map is generated to dispatch responding crews to the site.  The map should include

    - roads

    - man-made structures in the area

    - water sources and known vegetation

    - fire location

- Create Fire Polygon and Fire Line(s)

  - After crews arrive, they size up the fire and obtain an estimated boundary. This estimated boundary will make up the first Fire Polygon and Fire Line(s)

  - Obtain the fire origin, date, time, and cause. Typically, there are no assignment breaks early in the fire. Save information to the history feature dataset.

- Create IAP/Briefing Map

  - If the fire extends beyond the Initial Attack (IA) period, additional crews will be dispatched to the fire and the map will contain zymology for crew assignments.

- Post Fire Polygon to FTP.NIFC.GOV

  - ftp://ftp.nifc.gov/FTP_DIRECTIONS_INCIDENT_SUPPORT_v1.5.pdf

- Post Maps to FTP.NIFC.GOV

  - ftp://ftp.nifc.gov/FTP_DIRECTIONS_INCIDENT_SUPPORT_v1.5.pdf

- Post KMZ file to WWW.MNICS.ORG/wpress

- Maps to WWW.MNICS.ORG/wpress

- Create and Edit Data

  - At the start of each planning cycle, archive all previous data to the history feature dataset.

  - Start with a new map, and obtain the latest Fire Polygon, Perimeter Sector, Fire Line and Fire Point data.

  - Make changes to the Perimeter, any Labels, update any Fire Line and any Fire Point information. Update all maps.

- Create New Map Products

  - Fire Perimeter Map

  - Fire Progression Map

- Post new data to FTP.NIFC.GOV

  - ftp://ftp.nifc.gov/FTP_DIRECTIONS_INCIDENT_SUPPORT_v1.5.pdf

- Post new maps to FTP.NIFC.GOV

  - ftp://ftp.nifc.gov/FTP_DIRECTIONS_INCIDENT_SUPPORT_v1.5.pdf

- Post new KMZ file to WWW.MNICS.ORG/wpress

- Post new maps to WWW.MNICS.ORG/wpress

- Compress the Geodatabase

- Back up work for the planning period

- Exit ArcMap

DNRGARMIN

BACKGROUND

DNRGarmin was created to expedite the transfer of data between a computer and various models of Garmin GPS. The application was created by MN DNR and is free for anyone to use.

Download DNRGarmin

USE ON AN INCIDENT

DNRGarmin is:

- a “bridge” between GPS and computer

- a file conversion tool

DNRGarmin can be used with or without a GPS.

DNRGarmin can be used stand-alone or in conjunction with ArcMap, Landview, and/or Google Earth. The File | LOAD FROM and SAVE TO options will reflect what GIS programs are currently open.

There will always be three steps in using DNRGarmin

- Load data into DNRGarmin

- Edit data in DNRGarmin if necessary

- Save data out of DNRGarmin

Note that loading data into the bridge does not delete it from the source.

Instructions, troubleshooting, and listserve email group support are here

TIPS FOR GARMIN DAKOTA,OREGON, & OTHER NEW MODELS

DNRGarmin (current version 5.4.1) has not been updated to address slight changes in the data string format of newer Garmin GPS units. The transposition of a few field names, and the addition of a couple of fields cause errors when downloading both .GPX format waypoints and track data.

Errors may vary depending on the exact model of Oregon, Dakota, 60, and aviation GPS unit, but usually reference table records/fields when attempting a SAVE TO.

If using a Garmin Oregon, Dakota, 60, or other newer model, you’ll need to save downloaded data twice –the first time as a .TXT file and the second time as whatever format you wish. Saving the data as a .txt file will strip the “problem” fields from the dataset and render it 100% compatible with DNRGarmin.

.GPX waypoints from/for newer Garmin GPSes

- Connect the GPS to the computer and power it on.

- Open Windows Explorer and copy the desired GPX file from the GPS drive to elsewhere on the computer.

- Open DNRGarmin.

  - Go to File | Load from | File.

  - Select the file (make sure Type = .GPX) and click Open.

  - Go to File | Save to | File (make sure Type = .TXT).

  - Name the file and choose a saving place, then click OK.

  - Now go right back to File | Load From

  - Select the file you just saved and click Open.

  - Go to File | Save As and save to any format you choose.

Tracks from/for newer Garmin GPSes

- Connect the GPS to the computer and power it on.

- Open DNRGarmin.

  - Go to Track | Download

  - Go to File | Save to | File (make sure Type = .TXT).

  - Name the file and choose a saving place, then click OK.

  - Now go right back to File | Load From

  - Select the file you just saved and click Open.

  - Go to File | Save As and save to any format you choose.

THE FUTURE OF DNRGARMIN

In summer 2011 DNRGarmin is slated to be written as Open Source by a collaboration of natural resource agency programmers. This change hoped to expedite change requests and greatly enhance the functionality/compatibility of DNRGarmin with more GPS models.

GOOGLE EARTH/GOOGLE MAPS

BACKGROUND

While GE will not suffice as the primary GIS tool on an incident, it surpasses ArcMap and all other Spatial Data Viewers for:

- Ease and universality of use – anyone can install from www.earth.google.com

- Background matrix of recent imagery

- Creating maps to embed in a webpage

- KML and KMZ file formats are compatible with ArcGIS and DNRGarmin

LIMITATIONS TO USE

- Requires Internet access

- Only simple tools for data creation

- Background imagery poor in remote locations

- Completeness of local attractions/landmarks/resources

USE ON AN INCIDENT

When first assigned to an incident:

- Use Google Earth to create a location point of the incident .

  - Start Google Earth.

  - Pan and zoom to the incident location.

  - Click the Pushpin tool.

  - Add the Incident Name, symbol, description, etc, then click OK.

  - Right click and choose Save Place As. Save to a local drive.

  - Post the new file to www.mnics.org/wpress and ftp.nifc.gov.

During an incident:

- Create a KMZ file of the fire perimeter and post to Google Earth Community and the MIFC FTP site.

  - In ArcGIS create a Feature Class of the perimeter.

    - In ArcGIS use the Layer to KML tool to save the perimeter to filename.kmz file (note: not just selected features are saved –the entire feature class is saved).

  - In Windows Explorer right click the .KMZ and choose Open.

  - Post the exported KMZ file to www.mnics.org/wpress and ftp.nifc.gov at the end of each planning period.

- Create additional .KMZ files as needed using the steps above

To view a KMZ or KML file in Google Earth:

- Right-click the .KMZ or .KML filename and choose Open.

To view a KMZ or KML file in ArcGIS:

- Use the KML to Layer tool

WWW.MNICS.ORG

BACKGROUND

The software used to build and manage www.mnics.org/wpress  is WordPress. This Open Source application allows for collaborative changes to layout, content, and permissions from any internet-enabled device. We’re running a beta version of this website (thus the /wpress directory) while personnel learn the system the spring of 2011. In June 2011 the beta will become www.mnics.org.

The draft website resides on the same server with the same ISP as it did previously. Only the method for uploading products has changed.

The Dashboard is the management backside of the website, and is where pages and content are developed.

There are four levels of permission on the website, as shown in the diagram below:

WEBSITE ACCESS

Some pages and content may be password protected for MNICS member use only. That password is mnallrisk.

All GISS on an incident will have access to the Dashboard:

- An Administrator will set up an Incident Page on www.mnics.org/wpress  before a GISS arrives on an incident.

- An Administrator will also add new user accounts and set permissions.

- IMT GISS positions will be given Editor or Administrator Roles.

- To request new access go to http://mnics.org/wpress/?page_id=1597

- To change your password, get to the login page and click Lost your password?

- To request website changes at any time, click the webmaster link at bottom of any page

TRAINING

For website tip sheets and user manuals see http://mnics.org/wpress/?page_id=2472

For GIS SOPs, guides, and other GIS-specific documents on an incident, see http://mnics.org/wpress/?page_id=2689

NIFC FTP SITE

BACKGROUND

FTP.NIFC.GOV is the national FTP site for fire-based incidents. It is an integral part of the NWCG National GISS SOPs for sharing and storing incident data.

Whereas products for general public and media use are posted to www.mnics.org/wpress, the NIFC ftp site is intended as a “one stop shop” for agency personnel.

FTP.NFIC.GOV USE ON AN INCIDENT

- Log into the FTP site

All portions of the server are password protected. MNICS incidents can log into ftp.nifc.gov/Incident_Specific_Data/Eastern/Minnesota as

Username: 	Minnesota

Password:	MNallri$k

- Set up an Incident file structure on the FTP site

- At the onset of an incident, set up a new file structure as follows:

/Incident_Name/GIS

/base_data

/documents

/Farsite

/GPS

/incident_data

/products

/projects

/tools

/working

/others as needed

/ICS_Gen_Staff

/Documents

/IAPs

/others as needed

/Working

/Farsite

/others as needed

- Merge IAP files into a PDF

- When the IAP has been placed on the <INCIDENTDRIVE>: drive by the Resources Unit leader, merge the following into a PDF file called IncidentName_ IAP.PDF:

- IAP document

- IAP_Briefing_Map

- Fire_Perimeter_ Map

- Fire_Progression_Map

- Transportation_Map

- Save the merged  PDF in <IncidentDrive>:\Incident_Name\DOCS\YYYYMMDD\Incident_Name_IAP.pdf (DD is the day of the IAP Operational Period)

- Upload the merged PDF to the IAP Folder

- Upload  <INCIDENTDRIVE>:\Incident_Name\DOCS\YYYYMMDD\Incident_Name_IAP.pdf to ftp://ftp.nifc.gov/Incident_Specific_Data/EASTERN/Minnesota/Incident_Name/IAPs/

- Upload the merged PDF to the Incident_Name Folder

- Upload <INCIDENTDRIVE>:\Incident_Name\DOCS\YYYYMMDDDD\Incident_Name_IAP.pdf to ftp.nifc.gov/Incident_Specific_Data/EASTERN/Minnesota/Incident_Name/

- Notify MNICS and local agency personnel

- Send an email with the content below to:

- diane.nygaard@state.mn.us

- MIFCIntell.dnr@state.mn.us

- Local Host Agency Personnel

The IAP and briefing map have been uploaded to the federal FTP server at:

ftp://ftp.nifc.gov/Incident_Specific_Data/EASTERN/Minnesota/Incident_Name/

INCIWEB

BACKGROUND

Www.inciweb.org is the national ”clearinghouse” for public information about fire incidents.

On an incident the PIO will request an incident login and password from the Inciweb. The PIO then enters information about the Incident each planning period.

INCIWEB USE ON AN INCIDENT

GISS does not upload data or products to www.inciweb.org, though the site can act as a quick reference for current incidents nationwide. Only Incident PIOs are given a login on a per-incident basis. www.mnics.org/wpress has the Inciweb RSS feed posted on the right sidebar of each page.