| ## Plastic Sphere Dispenser Project Aviation Safety Plan |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Mission: Aerial Ignition, PSD | Project Name: Mad River Complex |  |  |  | Unit: Lassic Branch |  |
| Anticipated Project Date: 8/25-8/31 |  | Start Time: (as needed) |  | Ending Time: n/a |  |  |
| Project Plan Prepared by: Dean Bitterman |  |  | Title: Air Operations Branch Director |  |  | Date: 8/25/15 |
| Note: Signature by the preparer verifies that all personnel have the required training for the mission. Attach map, clearly showing areas to be flown; aerial hazards must be indicated. |  |  |  |  |  |  |
| Project Plan Reviewed by: |  |  | Title: AOBD-t |  |  | Date: |
| Project Plan Reviewed by: |  |  | Title: Burn Boss/FBAN |  |  | Date: |
| Project Plan Reviewed by: |  |  | Title: OSC1 |  |  | Date: |
| This Project is approved by: |  |  | Title: NR T1 IMT IC |  |  | Date: |

Project Description: Narrative: Limitations of topography and access, as well as volatile fuels and weather on the Lassic Fire present conditions where indirect line with burnout and firing from aerial ignition may be indicated. Aerial ignition will be utilized to facilitate fire spread in a controlled manner down to control lines, as well as remove unburned fuels between control lines and the main fire. Strategy: Aerial ignition will be utilized to mitigate potential high intensity fire and reduce fuels ahead of the fire. Firing sequence: (TBD by burn boss/Division Supervisor and Firing Boss). Generally ignition is conducted from the upslope/downwind edge of the area to be burned, progressing downslope/upwind. Specifics of the situation may require a deviation from this standard pattern, TBD by the burn boss/Division Supervisor and Firing Boss. The intent of this specific burn is to solidify the black in the Div Y/B interface fire ground such that reburn potential is significantly reduced in an effort to enhance firefighter safety as suppression efforts progress.

| Attachments: x Map x Aerial Ignition Checklist | x Other: (IAP and ICS-220 for day’s operation) |  |
| --- | --- | --- |
| Burn Boss: Tim Metzger | Phone: | Cell: 406- |
| Helicopter Manager: Kyle Talasono | Phone: | Cell: |
| PSD Operator: Kyle Talasono | Phone: | Cell: |
| ### Participants: Other personnel from Mad River Helibase, Dinsmore Helibase, as well as other helicopter module personnel may be utilized as appropriate for the operation. Specifics TBD and “pen and ink’d” as additions to this plan. |  |  |

| Type of Flight: Aerial Ignition, Sphere |  | Desired Make/Model: (not specified) |  |  |  | Charge Code: P5J0JC(0510) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type Procurement: Ex.Use |  |  |  | Method of Payment: |  |  | Projected Cost: $5000.ea |
| Vendor:Rogers Helicopters |  |  |  | Phone: |  |  | Cell: |
| Aircraft N#: N911VR | Make & Model: Bell 212 HP |  |  |  | Aircraft Color: Red/White |  |  |
| Pilot Name: |  |  |  | Pilot Carded: xYes No |  | A/C Carded: xYes No |  |
| Flight Follow: w/Helibase or aerial supervision on victor link |  |  |  | Request or Flight #: |  |  |  |
| Method of Resource Tracking: Phone Radio |  |  |  | Prior to Takeoff Each Stop En Route Arrival at Dest. |  |  |  |
| Scheduling Dispatch Phone: 707-726-1266 |  |  |  | Destination Dispatch Phone: |  |  |  |
| FM Receive: CMD: 168.100 |  |  | FM Transmit: 170.450 |  |  | Tones: 123.0 |  |
| FM Receive: A-G firing rx/tx:167.8875 |  |  | FM Transmit: |  |  | Tones: |  |
| FM Receive: A-G cmd rx/tx: 169.150 |  |  | FM Transmit: |  |  | Tones: |  |
| AM Air to Air: 126.45 (tfr) |  |  | AM Unicom: 122.8 (Ruth) |  |  | Other: |  |

Plastic Sphere Dispenser Project Aviation Safety Plan (continued)

| Passenger Name | Weight | Departure Point | Destination Point |
| --- | --- | --- | --- |
| (TBD—add to plan on day | of burn) |  |  |
| Cargo Weight | Cubic Feet of Cargo | Hazardous Material | Destination |
| PSD-100# |  | xYes No | Lassic Fire |
| Spheres: 17#/1000 spheres |  | Yes No |  |

| Crash/Search and Rescue Procedures: Contact Dispatch, follow local/regional crash/search and rescue guide/Aircraft Incident Response Plan. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Start Location | Latitude | Longitude | Elevation | Helibase/Helispot Size |
| Mad River Helibase | 40 12.476’N | 123 17.777’W | 2227 | airstrip |
| Destination Location | Latitude | Longitude | Elevation | Helibase/Helispot Size |
| H-310, Gel Plant | 40 17.603’ | 123 26.025’ | 3075’ | Open field, also Gel Plant |

| Type of Operation – check applicable boxes | Personal Protective Equipment Requirements |
| --- | --- |
| xHelo Ops – ground personnel | Nomex clothing, hardhat w/chin strap, gloves, leather boots, eye protection, hearing protection, fire extinguisher |
| xRotor Wing flights | Flight helmet, Nomex clothing, gloves, leather boots, eye protection, hearing protection, approved secondary restraint harness for doors off flights where pax is not fully within A/C |
| x Doors off flight where pax is not fully within aircraft | Use approved secondary restraint harness attached to approved aircraft hardpoints |

Justification statement for low-level flights:

Management has deemed aerial ignition as the best method of achieving Agency goals.  Aerial ignition is conducted below 500’ above ground level (AGL).  Reference IHOG chapter 3, Operational Planning.

Special instructions:

All PSD operations will be conducted in accordance with Manual and Handbook direction as well as the Interagency Aerial Ignition Guide and Interagency Helicopter Operations Guide.  A fire shelter will be available for the Pilot and trained in use.  Helicopter Manager, PLDO, and Parking Tender must be carded and current for PSD operations.  Helicopter Manager needs to assure that all the PSD equipment meets agency standards.

| # Aircraft Manager must confirm with Dispatch prior to the flight that affected routes’ Schedulers contacted for Route Activity # Military Training Route (MTR) Information (NA) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MTR | Route Legs-Altitude | Activity | Time |  | Time Zone |
|  |  | Hot Cold | Start | Stop | UTCLocal |
|  |  | Hot Cold | Start | Stop | UTCLocal |
|  |  | Hot Cold | Start | Stop | UTCLocal |
|  |  | Hot Cold | Start | Stop | UTCLocal |
|  |  | Hot Cold | Start | Stop | UTCLocal |
|  |  | Hot Cold | Start | Stop | UTCLocal |
|  |  | Hot Cold | Start | Stop | UTCLocal |
|  |  | Hot Cold | Start | Stop | UTCLocal |
|  |  | Hot Cold | Start | Stop | UTCLocal |

Plastic Sphere Dispenser Project Aviation Safety Plan (continued)

PSD Job Hazard Analysis

Aircraft Manager/Pilot review with all participants as part of preflight briefing.

| JOB HAZARD ANALYSIS (JHA) (Instructions on next page) |  | 1. WORK PROJECT/ACTIVITY |  | 2. LOCATION | 3. UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  | Plastic Sphere Dispenser Operations |  | Mad River Complex | 0510 |
|  |  | 4. PREPARED BY |  | 5. JOB TITLE | 6. DATE |
|  |  | Dean Bitterman |  | AOBD | 8/25/15 |
| 7. TASKS/HAZARDS | 8. ABATEMENT ACTIONS |  |  |  |  |
| Unqualified Personnel Unknown Responsibilities Aircraft Avoidance Weather High/Hot/Heavy Low level obstacles Doors off helicopter operations w/pax not fully enclosed in helicopter. Pilot not familiar with area Noise, rotor wash Unplanned aircraft events Hazardous materials Communications Rotor hazards Multiple project aircraft PSD Equipment Spheres/Gylcol Ignition Issues Aircraft Fueling Missing Aircraft, Crash/Search & Rescue PSD Malfunctions Cold Weather Operations | -Sphere Dispenser Operator shall be certified annually. Pilot and helicopter will be carded annually for PSD operations. Pilot will be knowledgeable in fire behavior and trained in use of the fire shelter. -Prior to each project, operator will review appropriate portions of IHOG and IAIG. The project briefing will cover responsibilities and emergency procedures -See and avoid. Check MTR routes in advance. Practice risk management; confirm that Dispatch has made contact with schedulers to de-conflict. Fly established airport patterns, initiate and stay in radio contact. TFR in place -Use weather advisory. Maintain VFR minimums, cancel mission if necessary. -Performance planning complete/insure accurate load calculations. Do not place the aircraft in performance related situations. -Complete a high level recon, no unnecessary low level flight. -Use approved secondary restraint harness in addition to seat belt. Remove/secure loose items from cabin. Know VNE. -Supply hazard maps. Complete high-level recon prior to low-level work, project area identified. -Wear ear and eye protection. -All personnel equipped with required PPE and trained in crash procedures. Review Crash Rescue/Medivac plan. Utilize Personnel Flotation Device when required. -Qualified personnel will handle, review MSDS, inform pilot. -Flight following established, checked and followed, communication plan posted. Maintain communications at all times, establish backup alternate frequencies. Take handheld radio along. Call in prior to landing. If radio contact is lost return to best suitable landing area and check-in. Parking tender outfitted with radio for takeoffs/landings. -Pilot perform aircraft safety brief, approach/depart safely or after shutdown and rotors stop. -Adequate aerial supervision. Carded managers for each aircraft. Maintain aircraft separation and positive communications. -Use only approved equipment with current retrofits as per IAIG. Bench testing will be completed prior to any operational mission and conducted a safe distance away from aircraft. -MSDS sheets on-site and reviewed, personnel briefed on hazards, transportation of hazmat complies with agency direction. -Conduct orientation flight with Ignition Specialist, hang fire mitigation and escaped fire contingency established, must complete all operational checklists prior to starting operations. - Vendor responsibility. No agency personnel on board. Aircraft shutdown unless closed circuit, open port in accordance with NFPA 407 3-21, 4073-21.2(b). Trained personnel staff extinguisher. - Duties assigned for extraction, suppression and flight following. Dispatch/helibase responsible to have current Aviation Incident Response/Crash SAR Plan posted and ready to implement. -Malfunctions will be addressed in project briefing. Operator will immediately notify pilot of problem and take appropriate action to correct. If malfunction cannot be corrected in the air, the helicopter will land. If fire occurs that the operator cannot extinguished, the pilot will be notified and appropriate action will be taken. -Utilize approved cold weather garments. This may include nomex hoods and winter weight nomex jackets, pants and flight suits. |  |  |  |  |
| 9. LINE OFFICER OR DESIGNEE SIGNATURE |  |  | 10. TITLE |  | 11. DATE |

Plastic Sphere Dispenser Project Aviation Safety Plan (continued)

PSD Job Hazard Analysis

| JHA Instructions The JHA shall identify the location of the work project or activity, the name of employee(s) writing the JHA, the date(s) of development, and the name of the appropriate line officer approving it. The supervisor acknowledges that employees have read and understand the contents, have received the required training, and are qualified to perform the work project or activity. Blocks 1, 2, 3, 4, 5, and 6: Self-explanatory. Block 7: Identify all tasks and procedures associated with the work project or activity that have potential to cause injury or illness to personnel and damage to property or material. Include emergency evacuation procedures (EEP). Identify all known or suspect hazards associated with each respective task/procedure listed in Block 7. For example: a. Research past accidents/incidents b. Research the Health and Safety Code, FSH 6709.11 or other appropriate literature. c. Discuss the work project/activity with participants d. Observe the work project/activity e. A combination of the above Block 8: Identify appropriate actions to reduce or eliminate the hazards identified in Block 8. Abatement measures listed below are in the order of the preferred abatement method: a. Engineering Controls (the most desirable method of abatement). For example, ergonomically designed tools, equipment, and Furniture. b. Substitution. For example, switching to high flash point, non-toxic solvents. c. Administrative Controls. For example, limiting exposure by reducing the work schedule; establishing appropriate procedures and practices. d. PPE (least desirable method of abatement). For example, using hearing protection when working with or close to portable machines (chain saws, rock drills portable water pumps) e. A combination of the above. Block 9: The JHA must be reviewed and approved by a line officer. Attach a copy of the JHA as justification for purchase orders when procuring PPE. Blocks 10 and 11: Self-explanatory. |  | Emergency Evacuation Instructions Project Supervisor and crew members are responsible for developing and discussing field emergency evacuation procedures (EEP) and alternatives in the event a person(s) becomes seriously ill or injured at the worksite. Be prepared to provide the following information: a. Nature of the accident or injury (avoid using victim's name). b. Type of assistance needed, if any (ground, air, or water evacuation) c. Location of accident or injury, best access route into the worksite (road name/number), identifiable ground/air landmarks. d. Radio frequency(s). e. Contact person. f. Local hazards to ground vehicles or aviation. g. Weather conditions (wind speed & direction, visibility, temp). h. Topography. i. Number of person(s) to be transported j. Estimated weight of passengers for air/water evacuation. The items listed above serve only as guidelines for the development of emergency evacuation procedures. |
| --- | --- | --- |
|  |  | JHA and Emergency Evacuation Procedures Acknowledgment We, the undersigned Project Supervisor and crew members, acknowledge participation in the development of this JHA (as applicable) and accompanying emergency evacuation procedures. We have thoroughly discussed and understand the provisions of each of these documents: |
| SIGNATURE DATE | SIGNATURE DATE |  |
| Project Supervisor |  |  |

### AVIATION RISK ASSESSMENT WORKSHEET (Example)

Assess the risks involved with the proposed operation.  Use additional sheets if necessary.    Line Officer/Designee Signature Required.  Reference IHOG Chapter 3.II.C.2

| Chart 3-2: Risk Assessment # Matrix |  |  | # HAZARD PROBABILITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Frequent | Likely | Occasional | Seldom | Unlikely |
|  |  |  | A | B | C | D | E |
| ## EFFECT | Catastrophic | I | Extremely High(4) |  |  |  | Medium |
|  | Critical | II |  |  | High(3) | Medium |  |
|  | Moderate | III | High | Medium(2) |  | Low(1) |  |
|  | Negligible | IV | Medium |  |  |  |  |

Assignment:																																															Date:

Pre-Mitigation hazards rate out as: High – 3.2

| Describe Hazard: | Probability (A-E) | Effect (I-IV) | Risk Level |
| --- | --- | --- | --- |
| 1. PSD malfunction-fire in machine attached to aircraft | C | II | HIGH 3 |
| 2. Doors off PSD machine jettison procedures-fall risk to operator. | E | I | MED 2 |
| 3. Aircraft malfunction | D | I | HIGH 3 |
| 4. General Aviation Aircraft in Area. Military Traffic in Area. Must be location specific | C | I | Ex HI 4 |
| 5. Over grossed aircraft/critical wind azimuth | D | I | Ex HI 4 |
| Mitigation Controls: Post-Mitigation hazards rate out as: Med – 1.8 | Probability (A-E) | Effect (I-IV) | Risk Level |
| 1. Emergency procedures covered with Pilot, Operator and Burn Boss/Ignition Specialist in pre-burn briefing. Water reservoir full in machine, pump operational. Spare gallon of water within reach of PSD operator. Machine clean. | D | II | MED 2 |
| 2. Utilize approved restraint harness and tether to approved attach point. | E | IV | LOW 1 |
| 3. Helicopter Manager ensures helicopter is carded and checks log books upon pre-use inspection. Pilot carded and trained in specific helicopter. | E | I | MED 2 |
| 4. Initiate and stay in radio contact with PSD Base. Monitor VHF. Have Dispatch check/clear MTRs in advance. Issue NOTAM if necessary. See and avoid. MUST be completed specific to project area. | E | I | MED 2 |
| 5. Pilot completes load calculation form properly for current and anticipated conditions. New load calculation will be completed for every 5  change (F) and 1,000’ elevation change. Manager will ensure load is commensurate with allowable payload as recorded on load calculation. | E | I | MED 2 |
| Approved By (Line Officer or Designee) Title: Date: |  |  |  |

Plastic Sphere Dispenser Project Aviation Safety Plan (continued)

Job Risk Analysis

Helicopter Manager/Pilot review with all participants as part of preflight briefing.

| Is everything approved with clear instructions, aviation plan signed and reviewed? | Yes | No | NA |
| --- | --- | --- | --- |
| Are communications and flight following established, including repeater tones? | Yes | No | NA |
| Can terrain, altitude, temperature, or weather that could have an adverse effect be mitigated? | Yes | No | NA |
| Are all aerial hazards identified and known to all participants? | Yes | No | NA |
| Have mitigating measures been taken to avoid conflicts with military or civilian aircraft. | Yes | No | NA |
| Have adequate landing areas been identified and or improved to minimum standards. | Yes | No | NA |
| Are all agency personnel qualified for the mission? | Yes | No | NA |
| Is the pilot carded and experienced for the mission to be conducted? | Yes | No | NA |
| Are pilot flight and duty times compromised? | Yes | No | NA |
| Are there enough agency personnel to accomplish the mission safely? | Yes | No | NA |
| Will adequate briefings be conducted prior to flight to include Pilot, Passengers, and Dispatch? | Yes | No | NA |
| Are all involved aware that the Pilot has the final authority, but if any passenger feels uncomfortable, that they can decline the flight? | Yes | No | NA |
| Is the aircraft capable of performing the mission with a margin of safety? | Yes | No | NA |
| Have manifests of cargo and passengers, load calculations, and/or weight and balance completed? | Yes | No | NA |
| Is the aircraft properly carded? | Yes | No | NA |
| Do all personnel have the required PPE? | Yes | No | NA |
| Fuel planning, adequate fuel on board, fuel truck location, availability of commercial fuel? | Yes | No | NA |
| Remember maps of areas/sites, handheld radios, cell phones, day/survival packs, and sick sacks. | Yes | No | NA |
| Is there an alternative method that would accomplish the mission more safely? | Yes | No | NA |
| Will the mission be conducted at low levels? (Below 500’ AGL). Discuss | Yes | No | NA |
| Can the same objectives be achieved by flying above 500’ AGL? | Yes | No | NA |
|  | Yes | No | NA |
| (minimize flight in height/velocity curve. Increase airspeed and altitude to degree practical. Brief with pilot the profile being requested, and adjust to safest profile as conditions/mission allows. | Yes | No | NA |
|  | Yes | No | NA |
|  | Yes | No | NA |
|  | Yes | No | NA |
|  | Yes | No | NA |
|  | Yes | No | NA |
|  | Yes | No | NA |