AIRCRAFT PRE-FIRE PLAN

AIRCRAFT TYPE: Aerospatiale Ecureuil Alstar AS 355	      			  Reference: T.O. 00-105E-9

AIRCRAFT DESCRIPTION: The AS 355 F-1 TwinStar is the twin engine version of the AS 350 B-2.

| Model | Length (ft) | Rotor Dia. (ft) | Bucket Size (gals) | 1.5 Hour Fuel Load (gallons) | Fuel Consumption (gal/hrs) | Cruise Speed (Kts/mph) | Typical Equip. Wt. (lbs.) | Fixed Wt. Reduction (lbs.) | Pax Seat | ICS Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 355 F-1 | 43 | 35 | 108-144 | 87 | 58 | 115 / 132 | 3250 | 140 | 4 | 3 |

AIRCRAFT DIAGRAM

# HAZARDS:

# Two Batteries are located inside of compartment, left side of fuselage.

# Fuel tanks are internally located, under cabin.

# Avoid main rotor and tail rotor.

# 268 Gallons / 1795 pounds of jet fuel.

# Tail rotor is located extremely close to the ground.

APPROACH TO ENTRY POINT:  (Engines running 80%+)

1.	Approach directly from the side, exercising extreme caution around rotors, which can dip to within four feet of the ground.

## PRE-DESIGNATED RESCUEMAN ACTIONS:

- GAIN ENTRY

a.	Normal entry –

I.	To open pilot’s door, turn door handle downward; door will open by itself.

II.	To open cabin door, turn door handle downward.

III.	Lightly pull door outward to release door from its housing, then push door backward.

b.	Internal Exiting –

I.	To open cabin door internally, turn top door bolt ¼ turn, then lower it.

II.	Turn bottom door bolt ¼ turn, then lift it.

c.	Forcible Entry/Cut- Cut around windows and door frames.

- ENGINE SHUTDOWN

a.	Push two fuel cut-off handles, located on overhead panel, sharply to AFT.

b.	Gently move two throttle handles, located on overhead panel, to the right while moving handles DOWNWARD and 			AFT.

c.	Press two master switch buttons, located on forward instrument panel, to the OFF position.

d.	Press two battery buttons, located on overhead panel, to the OFF position.

3.	AIRCREW EXTRACTION													a.	Release lap belt, shoulder harness, and any other restraints, then set restraint straps aside and remove 				crewmembers.

### OTHER FACTORS:

- Special tools/equipment: Power rescue saw and crash axe.

- Location of switches and equipment and emergency shutdown procedures may vary for individual aircraft.  Crews should be briefed prior to helicopter dispatch.

## COMMUNICATIONS PLAN: National Air Tanker Frequency – 123.975.  This is the frequency used by all air tanker bases, air tankers, and air tanker support aircraft during fire fighting operations.