AIRCRAFT PRE-FIRE PLAN

AIRCRAFT TYPE: Altus II						  Reference: Mfg. Info

AIRCRAFT DESCRIPTION: Altus II is an Unmanned Aerial Vehicle (UAV) designed for high-altitude scientific and commercial applications.  It is a derivative of the General Atomics Predator series of UAVs, currently being used by the military for surveillance.  Studies are currently being conducted to look into the possibility of using the aircraft to give Fire Incident Commanders real-time video of major brush-fires.  It is powered by a Rotax 914-2T dual turbo, liquid cooled, four cylinder engine rated at 100 horsepower at 52,000 feet.

| Wing Span | Length | Height | Cruise Speed | Range | Max. Weight | Loiter Time | Crew / PAX |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 55.3 ft | 23.6 ft | 9.8 ft | 100 KIAS | 125 m. | 2150 lbs | 24 hrs. | 0 |

AIRCRAFT DIAGRAM:

HAZARDS:

- Aircraft is made out of graphite epoxy.

- Aircraft has a single propeller mounted at the rear.

- Aircraft carries 550 pounds of 100 octane fuel.

- Aircraft can carry a parachute flight termination system that contains a rocket motor (0.74 lbs of Class 1.3C explosives) an igniter (0.220 grams of Class 1.4S explosives) and a cutter (0.065 grams of Class C explosives), located on the upper mid-section of the aircraft.

- Two standard aircraft grade dry-cell Nickel Cadmium (NiCad) batteries are located in the front avionics bay.

- Carries less than 3 gallons of standard automotive grade ethylene glycol anti-freeze and less than 3 gallons of SAE grade synthetic oil.

APPROACH TO ENTRY POINT:  Not applicable since aircraft is unmanned and remotely controlled.  If necessary to approach aircraft, avoid propeller at the rear of the aircraft.

PRE-DESIGNATED RESCUEMAN ACTIONS:

No rescue is necessary as aircraft is unmanned!

OTHER FACTORS:

- When involved in an accident, there is no need to risk fire fighters as aircraft is unmanned.

- Fire fighters should avoid smoke and dust from graphite epoxy.