AIRCRAFT PRE-FIRE PLAN

AIRCRAFT TYPE:	Beechcraft 58P Baron 				    	     Reference:

AIRCRAFT DESCRIPTION: The Forest Service owns and operates several 58P Barons.  Barons are light piston-powered airplanes, fully IFR capable and pressurized.  They are used as leadplanes and for passenger transportation.  They are maneuverable, fuel efficient, and can stay over an incident two to four hours, depending on weather and fire conditions.  Barons do not cruise as fast as some new generation airtankers such as the P-3A or the C-130.  These airtankers may reach an incident before the leadplane.  Each leadplane pilot has a specific designator; for example, Lead 1-5.  The first digit indicates the region the pilot is based at and the second digit indicates a specific pilot.

| Length (ft) | Wing Span (ft) | Cruise Speed (Kts/mph) | Range (St. miles) | Payload | Number of Pax Seats |
| --- | --- | --- | --- | --- | --- |
| 30 | 38 | 185 / 215 | 800 | 800 | 5 |

AIRCRAFT DIAGRAM

# HAZARDS

- 166 gallons / 1112 pounds of Aviation Gas on-board (fully loaded), located in wings and fuselage.

- Two batteries located in nose compartment.  (Prop hazard)  Press two buttons on handle, located on right side.  Handles will pop out.  Raise handles and raise hatch.  Battery located under floorboard.

- Propeller danger zone 25 feet

- Engine exhaust 100 feet.

APPROACH TO ENTRY POINT  (Engines running 80%+)

- Approach from right side, aft of wings avoiding the propellers for a ten-foot radius.

## PRE-DESIGNATED RESCUEMAN ACTIONS

- GAIN ENTRY

a.	Normal entry –

I.	MAIN PASSENGER CABIN: Rotate handle on door, located on right side of fuselage, aft of the wing, 	counterclockwise.  Swing door forward.

II.	COCKPIT: From a position on the right wing, press aft “bobbled” portion of handle to pop handle out.  PULL 		handle aft and swing door forward.

b.	Cut-In – Cut-in around large passenger windows on both sides.  These are emergency exits that can only be opened 			from the inside.

- ENGINE SHUTDOWN

a.	Retard Prop, Throttle and Mixture Levers, located lower center portion of dash, to the full aft and off positions.

b.	Turn two red fuel selectors, located on floor between pilots, to the OFF position.  (On the right switch this will be to 			the 3:00 position and on the left switch to the 9:00 position.)

c.	Press battery switch, located just above the left crewmember’s knee, between two alternator switches, down to OFF.  		(It is OK to press the two alternator switches also.)

3.	AIRCREW EXTRACTION

a.	Unlatch lap belts on all crewmembers and passengers.

### OTHER FACTORS

- Special tools/equipment: Power rescue saw and crash axe.

- Location of switches and equipment and emergency shutdown procedures may vary for individual aircraft.  Crews should be briefed prior to aircraft dispatch.

## COMMUNICATIONS PLAN: National Air Tanker frequency – 123.975.  This is the frequency used by all air tanker bases, air tankers, and air tanker support aircraft during fire fighting operations.