AIRCRAFT PRE-FIRE PLAN

AIRCRAFT TYPE: Bell 206 L-3 LongRanger III					        Reference: IHOG (1/98)

AIRCRAFT DESCRIPTION: The L-3 is a stretched variant of the Bell 206 B-III.  It was first designed as the L-1 LongRanger II.  The L-1 is the same size as the L-3 but performance is no better than B-IIIs.  The L-3 fills the gap between the small B-III and much larger 205.  It has a larger engine than the B-IIIs and at high elevations it performs better than some 205s.  Parts and service are readily available.  It is vry maneuverable and missions include bucket work, rappelling, recon, and personnel transport.  It is an excellent med-evac ship and is used by many air ambulance services.  Its extra length also allows it to carry stretchers.

| Length (ft) | Rotor Dia. (ft) | Bucket Size (gals) | 1.5 Hour Fuel Load (gallons) | Fuel Consumption (gal/hrs) | Cruise Speed (Kts/mph) | Typical Equip. Wt. (lbs.) | Fixed Wt. Reduction (lbs.) | Pax Seat | ICS Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43 | 37 | 96-144 | 52 | 35 | 107 / 127 | 2400 | 180 | 6 | 3 |

AIRCRAFT DIAGRAM

# HAZARDS

# Battery is located inside on nose of helicopter.  Remove front panel, and rotate knob counterclockwise, and disconnect cable from battery.

2.	Rotor blades.

# 3.	Fuel tanks are internally located.

APPROACH TO ENTRY POINT  (Engines running 80%+)

- Approach directly from the side, exercising extreme caution around rotors, which can dip to within four feet of the ground.

## PRE-DESIGNATED RESCUEMAN ACTIONS

- GAIN ENTRY

a.	Normal entry – Pilot and passenger doors have recessed handles pointing forward.  Outside – pull out and back.  Inside – pull up.

b.	Emergency entry – If cabin door or cargo passenger door fails to open, break windows or windshield.

c.	Forcible Entry/Cut-To gain entry to cabin, slide or break either cabin door window.

2.	ENGINE SHUTDOWN

a.	Rotate throttle, located on pilot’s collective pitch stick, to OFF position.

b.	Pull fuel shutoff valve, located overhead in crew compartment, aft to OFF position.

c.	Place battery switch, located on overhead switch panel, in OFF position.

3.	AIRCREW EXTRACTION

a.	Unlatch lap belts and remove shoulder harness from crewmember(s).

### OTHER FACTORS

- Special tools/equipment: Power rescue saw and crash axe.

- Location of switches and equipment and emergency procedures may vary for individual aircraft.  Crews should be briefed prior to helicopter dispatch.

## COMMUNICATIONS PLAN: National Air Tanker Frequency – 123.975.  This is the frequency used by all air tanker bases, air tankers, and air tanker support aircraft during fire fighting operations.