AIRCRAFT PRE-FIRE PLANS

NOTE: The following Aircraft Pre-Fire Plans are developed to assist Airport Rescue Fire Fighters (ARFF) in better preparing themselves for emergencies involving aircraft that frequent fixed-wing air attack bases and helibases.  It is the responsibility of each member of the ARFF crew to become familiar with the particular aircraft that they will be working with, as aircraft are modified differently by the various operators.  It must be stressed that these plans are a TRAINING TOOL and in no way are intended to replace current operating procedures and they shall not be considered as an operating procedure.

Any information or corrections should be sent to:

USFS – Fox Air Tanker Base

Attn: Tanker Base Manager

4555 West Avenue G, Suite 15

Lancaster, CA. 93536

TABLE OF CONTENTS

SECTION 1

GENERAL INFORMATION

Retardant Safety Information

Aircraft Not Covered In This Manual

Job Hazard Analysis

SECTION 2

INFRARED AIRCRAFT						        PLAN

Altus II	Altus II

ER-2	   ER-2

SECTION 3

LEAD PLANES	PLAN

Beechcraft 58P Baron	 Baron

SECTION 4

AIR ATTACK GROUP SUPERVISOR AIRCRAFT	PLAN

Cessna 337 Skymaster / O-2	 Cessna 337

Rockwell OV-10 Bronco	 OV-10

Beechcraft U-21 / King Air	   U-21

Aero Commander “Shrike”					       Commander

Turbo Commander					      Turbo Commander

Pilatus PC-7							       Pilatus PC-7

SECTION 5

SINGLE ENGINE AIR TANKERS (SEATS)			        PLAN

Grumman AgCat								        AgCat

Air Tractor AT-802							 Air Tractor

SECTION 6

AIR TANKERS								        PLAN

C-130 Hercules								        C-130

Canadair CL-215 / 415 Super Scooper				      CL-215

Convair 580							        Convair 580

DC-4 Airliner / C-54A, D, E, & G						DC-4

DC-6 Airliner / C-118								DC-6

DC-7 Airliner									DC-7

de Havilland DHC-2 Beaver / U-6					       DHC-2

de Havilland DHC-6 Twin Otter / UV-18				       DHC-6

Fokker F-27									 F-27

Boeing KC-97								        KC-97

Lockheed P2V Neptune / SP2H						   P-2

Lockheed P-3A Orion								   P-3

Consolidated PB4Y-2 Privateer					      PB4Y-2

Grumman S-2 Tracker								   S-2

SECTION 7

SMOKEJUMPER AIRCRAFT					        PLAN

Casa C-212 Aviocar							        C-212

Shorts SD 330 / C-23A Sherpa						 C-23

Douglas Turbine DC-3 / C-117C / BT-67					DC-3

SECTION 8

TRANSPORT AIRCRAFT						        PLAN Boeing 727									Boeing 727

Boeing 737									Boeing 737

DC-9 / MD-80 / 717 / C-9						       MD-80

SECTION 9

FOREIGN FIRE FIGHTING AIRCRAFT				        PLAN

Aeritialia G222 / C-27A							        C-27A Transall C-160								        C-160

SECTION 10

HELICOPTERS								        PLAN

Aerospatiale AS 350							      AS-350

Aerospatiale AS 355							      AS-355

Aerospatiale AS 332L Super Puma				    AS-332L

Bell 47									       Bell 47

Bell 204B									   Bell 204B

Bell 205									     Bell 205

Bell 206B									   Bell 206B

Bell 206L									   Bell 206L

AH-1P Fire Snake							       AH-1P

Bell 212 / 412 / UH-1N							     Bell 212

Bell 222									     Bell 222

BO-105									      BO-105

BV-107 / CH-46								      BV-107

BV 234 / CH-47								      BV 234

Kaman HH-43 Huskie							        HH-43

Hughes 500D / OH-6						        Hughes 500

Kaman K-Max Aerial Truck						        K-Max

CH-34 / S-58T								        S-58T

Sikorsky S-61N / SH-3 Sea King					          S-61

Sikorsky S-64 Skycrane / CH-54						 S-64

Sikorsky S-70 FireHawk / UH-60					        S-70A

SA-316B									    SA-316B

SECTION 11

Cross Reference of Aircraft Identifiers

SECTION 1

GENERAL INFORMATION

Retardant Safety Information

Aircraft Not Covered In This Manual

Job Hazard Analysis

RETARDANT HEALTH INFORMATION

Caution!  The following information is generated for the variety of retardants used at air tanker bases.  This is not intended to replace Material Safety Data Sheets (MSDS).  It is only to be used as a quick reference for training purposes and should not be considered as a reference for use during emergencies!

PRECAUTIONARY MEASURES:  Handle in accordance with good industrial hygiene and safety practices.  These practices include avoiding unnecessary contact and inhalation of dust, and removal from the eyes, skin and clothing.  Wear NIOSH approved respirator.

EYE CONTACT:  No more than slightly irritating based on toxicity studies.  The dry powder or liquid may cause foreign body irritation in some individuals.

EYE PROTECTION:  Wear chemical safety goggles to prevent eye contact.

SKIN CONTACT:  No more than slightly toxic based on toxicity studies.  None-irritating based on toxicity studies.  The dry powder or liquid may cause drying or chapping of the skin.

SKIN PROTECTION:  Wear protective gloves to minimize skin contact.  Wash hands and contaminated skin after handling.

INHALATION HAZARD:  Some retardants contain GUAR GUM which can cause an allergic respiratory reaction!  May cause respiratory tract irritation.  May cause nasal and respiratory tract irritation based on toxicity information of components.  Repeated or prolonged inhalation may cause an allergic reaction in some people.

RESPIRATORY PROTECTION:  Wear dust mask if dusty conditions exist.

INGESTION HAZARD:  Non-toxic if swallowed based on toxicity studies.  No significant adverse health effects are expected to develop if only small amounts (less than a mouthful) are swallowed.

FIRST AID MEASURES:  If in eyes, flush with plenty of water.  Wash skin and clothing after handling.  If inhaled, remove to fresh air.  If breathing is difficult, give oxygen.  If not breathing, perform artificial respiration.  If swallowed, immediate first aid is not likely to be required.  A Physician or Poison Control Center can be contacted for advice.

AIRCRAFT NOT COVERED IN THIS MANUAL

For the purpose of this manual, small aircraft which generally carry less than fifteen passengers will be considered General Aviation Aircraft.  This type of aircraft is not covered in this manual.  Wildland firefighting agencies utilize a variety of these aircraft to support fire fighting operations.  Some of the roles they play include observation and transportation of fire management personnel and equipment.

Fire Fighters assigned to air attack functions need to be aware that this type of aircraft may frequent their field.  Hazards associated with this type of aircraft include propellers and the possibility of hazardous materials in the baggage area.

Aircraft in this category can generally be treated as one would treat a vehicle accident or vehicle fire.  In most situations, incidents involving this type of aircraft can be handled by a single fire engine or Airport Rescue Fire Fighting (ARFF) apparatus.

In the case of larger commercial or transport aircraft, for the most part these will not be found at air attack bases.  However, occasionally the National Interagency Fire Center may charter a 727, 737 or MD-80 to transport a large number of crews to a particular incident.  In this case, the aircraft could fly into an air attack base.

An incident involving this type of aircraft will most definitely exceed the capabilities of a single fire apparatus.  Fire Officers need to consider, in advance, the additional resources that may be needed to deal with an incident involving one of these aircraft.

For more information on aircraft firefighting, please review the following resources:

1.	Aircraft Rescue Fire Fighting, 4th edition, International Fire Service Training Association.

2.	Federal Aviation Regulation 139. This regulation defines the requirements for Airport Rescue Fire Fighting (ARFF) services.

3.	National Fire Protection Association Codes.

4.	Interagency Helicopter Operations Guide (IHOG)

5.	US Air Force Technical Order 00-105E-9.  This is available on the internet through http://137.244.215.33/ti/tilta/documents/to00-105E-9.htm

SECTION 2

INFRARED AIRCRAFT						        PLAN

Altus II	Altus II

ER-2	   ER-2

Infrared aircraft are used by agencies such as NASA to photograph wildland fire incidents.  This information, when gathered, is then provided to fire managers to aid in developing tactics and strategies.  Photos are also taken using satellites.

SECTION 3

LEAD PLANES	PLAN

Beechcraft 58P Baron	 Baron

Lead planes are used to “lead” air tankers into a drop zone.  In most situations, these aircraft are flown by a single pilot.

SECTION 4

AIR ATTACK GROUP SUPERVISOR AIRCRAFT	PLAN

Cessna 337 Skymaster / O-2	 Cessna 337

Rockwell OV-10 Bronco	 OV-10

Beechcraft U-21 / King Air	   U-21

Aero Commander “Shrike”					       Commander

Turbo Commander					      Turbo Commander

Pilatus PC-7							       Pilatus PC-7

Air Attack Group Supervisor (ATGS) aircraft fly over a wildland incident.  Requests for air support are directed to the ATGS, who, acting like an airborne control tower, then assigns aircraft to the need.  The aircraft is flown by a pilot with a Fire Officer, who is knowledgeable in wildland firefighting and aircraft operations.

SECTION 5

SINGLE ENGINE AIR TANKERS (SEATS)			        PLAN

Grumman AgCat								        AgCat

Air Tractor AT-802							 Air Tractor

Single Engine Air Tankers, called SEATS, are basically agricultural aircraft, a.k.a. “Crop Dusters” that are utilized in wildland firefighting.  These aircraft can operate out of a tanker base or can land on remote roads close to the fire.

SECTION 6

AIR TANKERS								        PLAN

C-130 Hercules								        C-130

Canadair CL-215 / 415 Super Scooper				      CL-215

Convair 580							        Convair 580

DC-4 Airliner / C-54A, D, E, & G						DC-4

DC-6 Airliner / C-118								DC-6

DC-7 Airliner									DC-7

de Havilland DHC-2 Beaver / U-6					       DHC-2

de Havilland DHC-6 Twin Otter / UV-18				       DHC-6

Fokker F-27									 F-27

Boeing KC-97								        KC-97

Lockheed P2V Neptune / SP2H						   P-2

Lockheed P-3A Orion								   P-3

Consolidated PB4Y-2 Privateer					      PB4Y-2

Grumman S-2 Tracker								   S-2

SECTION 7

SMOKEJUMPER AIRCRAFT					        PLAN

Casa C-212 Aviocar							        C-212

Shorts SD 330 / C-23A Sherpa						 C-23

Douglas Turbine DC-3 / C-117C / BT-67					DC-3

There are a variety of aircraft that can be used to transport Smokejumpers to an incident.  These aircraft can haul one or two jumpers up to the larger aircraft, detailed here, which can transport a whole crew.

SECTION 8

TRANSPORT AIRCRAFT						        PLAN Boeing 727									Boeing 727

Boeing 737									Boeing 737

DC-9 / MD-80 / 717 / C-9						       MD-80

These aircraft are occasionally chartered by the National Interagency Fire Center (NIFC) in Boise, Idaho to transport large numbers of fire fighters cross country to major incidents.  The three aircraft detailed here are the most commonly used, with the 737 flown by Sierra Pacific Airlines the most popular.

SECTION 9

FOREIGN FIRE FIGHTING AIRCRAFT				        PLAN

Aeritialia G222 / C-27A							        C-27A Transall C-160								        C-160

The State Department, in the past, has received request from foreign countries for assistance in combating large vegetation fires.  Because of the remote possibility of personnel coming into contact with aircraft from other countries, Plans are included on several foreign aircraft.

It should also be noted, that occasionally, aircraft from Canada may be contracted to fly incidents in the US.

SECTION 10

HELICOPTERS								        PLAN

Aerospatiale AS 350							      AS-350

Aerospatiale AS 355							      AS-355

Aerospatiale AS 332L Super Puma				    AS-332L

Bell 47									       Bell 47

Bell 204B									   Bell 204B

Bell 205									     Bell 205

Bell 206B									   Bell 206B

Bell 206L									   Bell 206L

AH-1P Fire Snake						                 AH-1P

Bell 212 / 412 / UH-1N							     Bell 212

Bell 222									     Bell 222

BO-105									      BO-105

BV-107 / CH-46								      BV-107

BV 234 / CH-47								      BV 234

Kaman HH-43 Huskie							        HH-43

Hughes 500D / OH-6						        Hughes 500

Kaman K-Max Aerial Truck						        K-Max

CH-34 / S-58T								        S-58T

Sikorsky S-61N / SH-3 Sea King					          S-61

Sikorsky S-64 Skycrane / CH-54						 S-64

Sikorsky S-70 FireHawk / UH-60					        S-70A

SA-316B									    SA-316B

SECTION 11

Cross Reference of Aircraft Identifiers

A particular model of aircraft can be known by a variety names depending on whether ones background is in the civilian aviation sector or a particular branch of the military.  What follows is a cross reference listing the majority of names that a particular aircraft may be known by and which Aircraft Pre-Fire Plan has information on that aircraft.

| INDEX OF AIRCRAFT |  |  |
| --- | --- | --- |
| AIRCRAFT PRE-FIRE PLANS |  |  |
| TYPE | SEE | SECTION |
| 47 | Bell 47 | 10 |
| 204 | Bell 204A | 10 |
| 212 | Bell 212 | 10 |
| 222 | Bell 222 | 10 |
| 412 | Bell 212 | 10 |
| 580 | Convair 580 | 6 |
| 717 | MD-80 | 8 |
| 727 | Boeing 727 | 8 |
| 737 | Boeing 737 | 8 |
| 204 B+ | Bell 204A | 10 |
| 205 A+ | Bell 205 | 10 |
| 205 A++ | Bell 205 | 10 |
| 206 B-3 JetRanger | Bell 206B | 10 |
| 206 L-3 LongRanger | Bell 206L | 10 |
| 214 B-1 | Bell 205 | 10 |
| 4Y | PB4Y-2 | 6 |
| 500D | Hughes 500 D | 10 |
| 530F | Hughes 500 D | 10 |
| 58P | Baron | 3 |
| Aerial Truck | K-Max | 10 |
| Aeritalia (Alena) G222 | C-27A | 9 |
| Aero Commander 500S | Commander | 4 |
| Aero Commander 690B | Turbo Commander | 4 |
| Aerospatiale AS 350 D-1 A-Star | AS-350 | 10 |
| Aerospatiale AS 355 F-1 Twin Star | AS-355 | 10 |
| Aerospatiale AS-332L Super Puma | AS-332L | 10 |
| Aerospatiale AS-350 B-2 Ecureuil | AS-350 | 10 |
| Aerospatiale SA-315B Lama | SA-315B | No info |
| Aerospatiale SA-316B Alouette III | SA-316B | 10 |
| Air Tractor AT 802 | Air Tractor | 5 |
| Alena G222 | C-27A | 9 |
| Alouette III | SA-316B | 10 |
| Altus II | Altus II | 2 |
| AS-332L Super Puma | AS-332L | 10 |
| AS-350 B-2 Ecureuil | AS-350 | 10 |
| AS-350 D-1 A-Star | AS-350 | 10 |
| AS-355 F-1 Twin Star | AS-355 | 10 |
| A-Star | AS-350 | 10 |
| AT 802 | Air Tractor | 5 |
| Aviocar | C-212 | 7 |
| Baron | Baron | 3 |
| BE 234 | BV-234 | 10 |
| Beaver | DHC-2 | 6 |
| Beechcraft 58P | Baron | 3 |
| Beechcraft King Air | U-21 | 4 |
| Beechcraft U-21 | U-21 | 4 |
| Bell 204B | Bell 204B | 10 |
| Bell 205A-1 | Bell 205 | 10 |
| Bell 206 B-3 JetRanger | Bell 206B | 10 |
| Bell 206 L-3 LongRanger | Bell 206L | 10 |
| Bell 209 | AH-1P | 10 |
| Bell 212 | Bell 212 | 10 |
| Bell 214 B-1 | Bell 205 | 10 |
| Bell 222 | Bell 222 | 10 |
| Bell 412 | Bell 212 | 10 |
| Bell 47 Soloy | Bell 47 | 10 |
| Bell Super 204 (204 B+) | Bell 204B | 10 |
| Bell Super 205 (205 A+) | Bell 205 | 10 |
| Bell Super Duper (205 A++) | Bell 205 | 10 |
| BK-117 | BK-117 | No info |
| Black Hawk | S-70 | 10 |
| BO-105 | BO-105 | 10 |
| Boeing 717 | MD-80 | 8 |
| Boeing 727 | Boeing 727 | 8 |
| Boeing 737 | Boeing 737 | 8 |
| Boeing BE 234 | BV-234 | 10 |
| Boeing BV 107 | BV-107 | 10 |
| Boeing KC-97 | KC-97 | 6 |
| Boeing Model 367 | KC-97 | 6 |
| Bombardier CL-215 | CL-215 | 6 |
| Bombardier CL-415 | CL-215 | 6 |
| Bronco | OV-10 | 4 |
| BT-67 | DC-3 | 7 |
| BV-107 | BV-107 | 10 |
| BV-234 | BV-234 | 10 |
| C 160 | C 160 | 9 |
| C-117C | DC-3 | 7 |
| C-118 | DC-6 | 6 |
| C-130 | C-130 | 6 |
| C-212 | C-212 | 7 |
| C-23A | SD 330 | 7 |
| C-27A | C-27A | 9 |
| C-47 | DC-3 | 7 |
| C-54 | DC-4 | 6 |
| C-9 Nightingale | MD-80 | 8 |
| Canadair CL-215 | CL-215 | 6 |
| Canadair CL-415 | CL-215 | 6 |
| Casa C-212 Aviocar | C-212 | 7 |
| Cayuse | Hughes 500 D | 10 |
| Cessna 337 | Cessna 337 | 4 |
| CH-3 | S-61 | 10 |
| CH-34 | S-58T | 10 |
| CH-46 | BV-107 | 10 |
| CH-47 | BV-234 | 10 |
| CH-54 | S-64 | 10 |
| Chinook | BV-234 | 10 |
| Choctaw | S-58T | 10 |
| CL-215 | CL-215 | 6 |
| CL-415 | CL-215 | 6 |
| Cloudmaster | DC-6 | 6 |
| Cobra | AH-1P | 10 |
| Commander 500S | Commander | 4 |
| Commander 690B | Turbo Commander | 4 |
| Consolidated PB4Y-2 Privateer | PB4Y-2 | 6 |
| Convair 580 | Convair 580 | 6 |
| Dakota | DC-3 | 7 |
| DC-3 | DC-3 | 7 |
| DC-4 | DC-4 | 6 |
| DC-6 | DC-6 | 6 |
| DC-7 | DC-7 | 6 |
| DC-9 | MD-80 | 8 |
| de Havilland DHC-2 Beaver | DHC-2 | 6 |
| de Havilland DHC-6 Twin Otter | DHC-6 | 6 |
| Defender 500 | Hughes 500 D | 10 |
| DHC-2 | DHC-2 | 6 |
| DHC-6 | DHC-6 | 6 |
| Douglas DC-3 | DC-3 | 7 |
| Douglas DC-4 | DC-4 | 6 |
| Douglas DC-6 | DC-6 | 6 |
| Douglas DC-7 | DC-7 | 6 |
| Douglas DC-9 | MD-80 | 8 |
| Douglas Turbine DC-3 | DC-3 | 7 |
| DragonLady | ER-2 | 2 |
| Ecureuil | AS-350 | 10 |
| ER-2 | ER-2 | 2 |
| Erickson Skycrane | S-64 | 10 |
| Eurocopter (MBB) BO-105 CB | BO-105 | 10 |
| Eurocopter BK 117 A-4 | BK-117 | No info |
| F-27 | F-27 | 6 |
| Fairchild F-27 | F-27 | 6 |
| Fairchild Hiller (FH) 1100 | FH-1100 | No info |
| FH-1100 | FH-1100 | No info |
| Fire Snake | AH-1P | 10 |
| FireCat | S-2 | 6 |
| FireHawk | S-70 | 10 |
| Fokker F-27 | F-27 | 6 |
| Four Y | PB4Y-2 | 6 |
| Friendship | F-27 | 6 |
| Grumman S-2F Tracker | S-2 | 6 |
| H-1 | See Bell | 10 |
| H-13 | Bell 47 | 10 |
| H-3 | S-61 | 10 |
| H-34 | S-58T | 10 |
| H-43 | H-43 | 10 |
| Hercules | C-130 | 6 |
| HH-1H | Bell 205 | 10 |
| HH-3 | S-61 | 10 |
| HH-43 | H-43 | 10 |
| HH-60 | S-70 | 10 |
| Hiller 12 D/E Soloy | Hiller 12 | No info |
| Hiller FH-1100 | FH-1100 | No info |
| Huey | See Bell | 10 |
| Hughes 500D | Hughes 500 D | 10 |
| Huskie | H-43 | 10 |
| Iroquois | See Bell | 10 |
| JetRanger | Bell 206B | 10 |
| Jolly Green Giant | S-61 | 10 |
| Kaman Aerial Truck | K-Max | 10 |
| Kaman H-43 Huskie | H-43 | 10 |
| KC-97 | KC-97 | 6 |
| King Air | U-21 | 4 |
| Kiowa | Bell 206B | 10 |
| K-Max | K-Max | 10 |
| Lama | SA-315B | No info |
| Lockheed C-130 | C-130 | 6 |
| Lockheed C-27A | C-27A | 9 |
| Lockheed ER-2 | ER-2 | 2 |
| Lockheed P-2 Neptune | P-2 | 6 |
| Lockheed P-3 Orion | P-3 | 6 |
| LongRanger | Bell 206L | 10 |
| MBB BO-105 | BO-105 | 10 |
| McDonnel Douglas MD 500D | Hughes 500 D | 10 |
| McDonnel Douglas MD 530F | Hughes 500 D | 10 |
| McDonnell-Douglas MD-80 | MD-80 | 8 |
| MD 500D | Hughes 500 D | 10 |
| MD 530F | Hughes 500 D | 10 |
| MD-80 | MD-80 | 8 |
| Model 367 | KC-97 | 6 |
| Model 90 | U-21 | 4 |
| Neptune | P-2 | 6 |
| Nightingale | MD-80 | 8 |
| North American OV-10 | OV-10 | 4 |
| O-2A | Cessna 337 | 4 |
| OH-5 | FH-1100 | No info |
| OH-6 | Hughes 500 D | 10 |
| Orion | P-3 | 6 |
| OV-10 | OV-10 | 4 |
| P-2 | P-2 | 6 |
| P2V-5 | P-2 | 6 |
| P2V-7 | P-2 | 6 |
| P-3 | P-3 | 6 |
| PB4Y-2 | PB4Y-2 | 6 |
| PC-7 | Pilatus PC-7 | 4 |
| Pedro | H-43 | 10 |
| Pilatus PC-7 | Pilatus PC-7 | 4 |
| Predator | Altus II | 2 |
| Privateer | PB4Y-2 | 6 |
| Rockwell OV-10 | OV-10 | 4 |
| Rogerson Hiller 12 | Hiller 12 | No info |
| RQ-4 | Altus II | 2 |
| S-2 | S-2 | 6 |
| S-58T | S-58T | 10 |
| Schweizer AgCat | AgCat | 5 |
| SD-330 | SD 330 | 7 |
| Sea King | S-61 | 10 |
| Sea Knight | BV-234 | 10 |
| Seven Seas | DC-7 | 6 |
| SH-46 | BV-107 | 10 |
| SH-60 | S-70 | 10 |
| Sherpa | SD 330 | 7 |
| Shorts SD-330 | SD 330 | 7 |
| Shrike | Commander | 4 |
| Sikorsky S-58T | S-58T | 10 |
| Sikorsky S-61 | S-61 | 10 |
| Sikorsky S-64 | S-64 | 10 |
| Sikorsky S-70 | S-70 | 10 |
| Skycrane | S-64 | 10 |
| Skymaster | Cessna 337 | 4 |
| Skymaster | DC-4 | 6 |
| Slick (UH-1) | See Bell | 10 |
| Soloy, Bell 47 | Bell 47 | 10 |
| Soloy, Hiller 12 D/E | Hiller 12 | 10 |
| SP2H | P-2 | 6 |
| Startocruiser | KC-97 | 6 |
| Startofreighter | KC-97 | 6 |
| Super 204 B+ | Bell 204B | 10 |
| Super 205 A+ | Bell 205 | 10 |
| Super Convair | Convair 580 | 6 |
| Super Duper 205 A++ | Bell 205 | 10 |
| Super PB4Y-2 | PB4Y-2 | 6 |
| Super Puma | AS-332L | 10 |
| Super Scooper | CL-215 | 6 |
| Super Skymaster | Cessna 337 | 4 |
| Tarhe | S-64 | 10 |
| TR-1 | ER-2 | 2 |
| Tracker | S-2 | 6 |
| Transall | C 160 | 9 |
| Turbo Commander | Turbo Commander | 4 |
| Turbo Tracker | S-2 | 6 |
| Twin Huey | Bell 212 | 10 |
| Twin Otter | DHC-6 | 6 |
| Twin Star | AS-355 | 10 |
| U-2 | ER-2 | 2 |
| U-21 | U-21 | 4 |
| U-6 | DHC-2 | 6 |
| UH-1A | Bell 204B | 10 |
| UH-1B | Bell 204B | 10 |
| UH-1D | Bell 205 | 10 |
| UH-1H | Bell 205 | 10 |
| UH-1N | Bell 212 | 10 |
| UH-60 | S-70 | 10 |
| Ute | U-21 | 4 |
| UV-18 | DHC-6 | 6 |