___________INCIDENT

# BURN PLAN

| Prepared by: | Date: |
| --- | --- |

# INTRODUCTION

This Firing Plan is being developed for implementation in support of on-going control efforts. This plan will incorporate favorable firing conditions as suggested by local fire management personnel who are familiar with the terrain and fuels in the local area, and input from the Fire Behavior Analyst's assigned to _________Incident. The Plan will give general guidance for each Division where firing operations are conducted. Specific direction and timing of firing operations will be coordinated from Operations through the Firing Coordinators which will normally be the Division Supervisors or others if specified. It will address Communications and foremost it will address Safety. The Operations Sections will provide final review and recommendations for acceptance of the firing plan. Coordination between the team members and adjoining divisions will be critical for successfully implementing part or all of this Firing Plan. If concurrent firing operations are planned on divisions in the general vicinity of each other, the Operations Sections will set priorities to ensure a safe firing operation can be conducted.

Administrative Unit:

Burn Location (see attached Map):

Fuel Model:

Fuel Arrangement/Continuity:

Slope:	 		Position on Slope:			Aspect:

Elevation:

Resource Values:

Public Safety:

Structures Involved:

T&E Species:

Mitigation Measures:

#### Objectives:

##### 1.

2.

##### FIRING PREPARATION

| Line to Construct |  |
| --- | --- |
| Helispots to Construct |  |
| Planned Completion Date | As weather and fuel moisture permit. |

##### BURN ORGANIZATION

| POSITION | ### Primary | Alternate |
| --- | --- | --- |
| Burn Boss |  |  |
| Firing Specialist – Ground |  |  |
| Firing Specialist – Ground |  |  |
| Firing Specialist – Aerial |  |  |
| Holding Specialist - Division D |  |  |
| Holding Specialist - Division Z |  |  |
| Holding Specialist - Division C |  |  |
| Fire Behavior Analyst |  |  |

Firing Pattern/Plan: (Consider desired fire behavior characteristics and relative location of active head/flanks of the fire prior to firing.  Provide opportunity for test fire and attach a map.)

Hand firing will occur when ever possible along all boundaries to provide a black line to increase the chances of success.  Aerial ignition will be used to add depth to the black line to further increase the chances of success and secure the fire.

Ignition patterns will be determined by the DIVS, ignition specialist and/or Burn Boss and coordinated to all key personnel involved in the burn operations.  The burn pattern can be modified as determined by the burn boss due to changes in the weather or fire behavior to achieve the objectives of the burn.

Equipment Needed For Firing Operation:

| FIRING EQUIPMENT | ## QUANTITY |
| --- | --- |
| Drip Torches + fuel | 4 ea. 20 gal fuel ea. |
| Flares | 1 Case |
| PSD + spheres and back up PSD | 10,000  spheres |
| Helicopter(s) | 3 (1 – ignition, 1 – bucket support, 1 - evacuation) |

Trigger Points (identify any conditions that might be trigger points for altering the plan as implemented):

Implement burnout operations when wind is favorable to maintain control.  If the wind switches to an unfavorable direction stop firing operations.

If T-Storms develop in the area stop firing operations until the storms pass the area.

Holding Operation:

| HOLDING RESOURCES | ## QUANTITY |
| --- | --- |
| Crews |  |
| Engines |  |

COMMUNICATIONS PLAN:

Assigned radio frequencies for the hand firing operation will be consistent with the assigned Division frequencies.

| DIVISION | TACTICAL CH. | FREQUENCY | COMMAND CH. | FREQUENCY |
| --- | --- | --- | --- | --- |
|  | Division Tac | See ICS 205 | See ICS 205 | See ICS 205 |
|  | Division Tac | See ICS 205 | See ICS 205 | See ICS 205 |
|  | Division Tac | See ICS 205 | See ICS 205 | See ICS 205 |

|  | PRIMARY | SECONDARY |
| --- | --- | --- |
| AERIAL IGNITION DISCRETE FREQUENCY |  |  |
| AIR TO GROUND- FM | See ICS 205 |  |
| FLIGHT FOLLOWING | See ICS 205 |  |
| AIR TO AIR | See ICS 205 |  |

Prior to aerial ignition being implemented a discrete frequency will be assigned to the aerial ignition operation. This frequency will be known to all ground forces with instructions to not use the frequency except for emergencies situations.

Notification Checklist:

Prior to implementing firing operations all those identified in the firing operations plan will ensure everyone is using the assigned radio frequencies. The following checklist will be used prior to each firing operation to ensure all resources assigned or responsible have been notified of the firing operation.

| POSITION | ### NOTIFIED- YES | NO |
| --- | --- | --- |
| Firing Burn Boss |  |  |
| Firing Specialist(s) |  |  |
| Firing Crew(s) |  |  |
| Ignition Specialist(s) |  |  |
| Plastic Sphere Operator |  |  |
| Holding Crew(s) |  |  |
| Communications Unit(s) |  |  |
| Air Operations Branch Dir. |  |  |
| Operations Section Chief(s) |  |  |
| Division/Group Supervisor(s) |  |  |

Safety Plan:

A thorough safety briefing with all resources involved will be conducted prior to implementation of this firing operation. Validation of LCESs will be briefed. Periodic revalidation of the LCESs will be done by the Firing Burn Boss.  The assigned Division Safety Officers will be identified during the briefing.

Additional Safety Instructions:

##### Contingency Plan:

##### Notify the Branch Director of any problems and an appropriate management response will be coordinated through Line Operations.

##### Local Coordination and Public Awareness:

Fire Information Officer, adjacent divisions and lookouts will be notified of burn plan and expected ignitions.

Final Outcome (describe the success of the burnout):

Indirect fire lines are secured and all unburned material is consumed.  The main areas of concern are along the main travel ways and around structures so public safety from smoke and fire is reduced.

Reviewed by: ______________________ Title _____________________  Date: ________

Reviewed by: ______________________ Title _____________________  Date: ________

Approved by: ______________________ Title: _____________________ Date: _________

____________FIRE

AERIAL IGNITION

BURN PLAN

PREMO MK III SPHERE DISPENSER

NORTHERN ROCKIES

INICIDENT MANAGEMENT TEAM

| AGENCY UNIT: | DATE: |
| --- | --- |

Prepared by: Date: AOBD/ASGS

Reviewed by: Date: PSDO/Helitorch Mgr.

Reviewed by: Date: OSC1

Reviewed by: Date: Unit Aviation Officer

Approved by: Date: Incident Commander

This burn plan in conjunction with the Air Operations Plan in Appendix A contains all nine requirements of the air operations portion of a prescribed burning plan as required in FSM 5153.1 and as described in the Interagency Aerial Ignition Guide. Approval of this plan by both the Incident Commander and Air Operations Branch Director constitutes the authority to burn while using an aerial ignition system.

APPENDIX A

I. OVERVIEW

Objectives

This Air Operations Plan is to be used as an integral part of the ___________Wildland Fire Plan for the                            Burn. The primary objective of this plan is to assure a safe and efficient Aerial Ignition Operation. It shall be used in conjunction with an approved plastic sphere dispenser as described in the Interagency Aerial Ignition Guide, Chapter III.

II BURN AREA FLIGHT HAZARDS

Flight patterns shall be planned to avoid overflights of buildings and structures in the area when the PSD is on the helicopter. The helicopter will maintain an elevation of at least 150 feet above the treetops and other obstacles during recon and ignition flights.

NORTH

NOT TO SCALE

LOCAL HAZARDS

1.

2.

3.

4.

IIl. AIR OPERATIONS ORGANIZATION:

A. PERSONNEL ASSIGNMENTS

Pilot(s)

Helibase Manager

Helibase Flight Following Air Attack on VHF – AM ____________This is the same as TFR ___________ Alternate Flight Following will be Big Cypress Fire Dispatch

IV. AIRCRAFT INFORMATION

A. HELICOPTER

Helicopter FAA No. N

Make(s) and Type(s) of Aircraft

Vendor Name(s) and Address

B. SPHERE DISPENSER(S)

Model(s) Premo Mark III

C. FIXED WING AIRCRAFT

Aircraft FAA No. N

Make(s) and Type(s) of Aircraft

Vendor Name(s) and Address

V. HELICOPTER FLIGHT

A. RECON FLIGHT

Prior to ignition a recon flight will be taken to establish the ignition pattern. If this is a Plastic Sphere Dispenser burn, the Burn Boss and PSLO shall be on this flight. During the recon flight, the proposed burn area will be thoroughly checked for any people and or wildlife that might be in the area and threatened by the burning operation. If people are present within the burn area boundaries, the Burn Boss will make a no go decision on ignition until all people are confirmed to be clear of the burn area. Other personnel, such as holding, ignition or observers shall be briefed, in radio communication and clear of the drop zone for spheres.

B. IGNITION FLIGHT

Although the ignition pattern will be directed by the Firing Boss, the helicopter pilot has final authority for flight safety and decisions as to which flight maneuvers may be conducted safely.

Vl. COMMUNICATIONS

Helicopter mission crewmembers and pilot shall have interphone capability and VHF-FM air-to-ground radio contact at all times. In addition, the pilot will monitor the appropriate VHF-AM frequency- which may be ___________ or ATC. During a wildland fire operation, the helispot mgr., helibase, or ATGS may function as operational radio contact. In the event of loss of radio capability, the firing operation will cease and the helicopter will immediately return to the helispot/ helibase.

A. FREQUENCIES

In order to minimize radio traffic on a division/ branch frequency, a discrete air-to-ground communications will be conducted on VHF-FM   ____________. Communication with helibase will be the Command Repeater     _______________        .  The helicopter will flight follow every 15 minutes with    _________on frequency VHF-AM      __________. The helispot mgr., helibase, and ATGS will monitor VHF-AM          ________for any possible civilian aircraft in the burn area.

B. COMMUNICATION CHART-  WILDLAND FIRE PSD OPERATION

Operations  Section Chief or

Branch Director

Command Repeater

Rx Burn Boss  1

Ignition Specialist 1

PSD Operator

Helicopter Pilot

Ignition A/G

Ignition Air – Ground

Division Group Supervisor

Strike Team/Task Force Leaders

Crew Bosses

Flight Following

Flight Following

Ignition Air – Ground

Air Tactical Group Supervisor

Other Incident A/C

Helibase Mgr./

Radio Operator

Air Guard should be used as an emergency or initial contact.

168.6250 RX / 168.6250 TX Tone 110.9

VIl. HELIBASE(s)  LOCATION

A. HELIBASE

| NAME | LOCATIONS- T/R/S | LAT/LONG |
| --- | --- | --- |
| ELEVATION | DUST ABATEMENT | DESCRIPTION |

B. HELISPOTS

In addition to the main helibase, other possilbe emergency helispots occur within the proposed burn area. These helispots can be used as emergency landing spots for the helicopter. These spots will be identified and approved by the pilot during the recon flight. Their location and names will be identified to the helibase radio operator.

| NAME | LOCATIONS- T/R/S | LAT/LONG |
| --- | --- | --- |
| ELEVATION | DUST ABATEMENT | DESCRIPTION |

| NAME | LOCATIONS- T/R/S | LAT/LONG |
| --- | --- | --- |
| ELEVATION | DUST ABATEMENT | DESCRIPTION |

VIII CRASH RESCUE

A. READY ALERT AIRCRAFT

Refer to the Incident Action Plan, Air Operations Summary ICS 220 for the date of the burn to identify the ready alert aircraft. The Helibase Manager will implement the crash rescue plan posted at the helibase. The Helibase Manager or his/her designee will coordinate the crash rescue activities.

B. COMMERCIAL MEDIVAC HELICOPTERS

Refer to to the Incident Action Plan, Medical Plan ICS 206 for the date of the burn to identfy commercial medivac helicopters and the procedure for implementing Emergency Medical Procedures.

C. EMERGENCY HELISPOT

One helispot manager and one helitack will be staged at the emergency helispot along with a fire extinguisher and crash rescue kit.

PSD OPERATIONS CHECKLIST

| PLASTIC SPHERE DISPENSER CHECKLIST |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| HELIBASE MANAGER |  |  |  |  | DATE |
| BURN BOSS/FIRING BOSS |  |  |  |  | DATE |
| DAILY INSPECTION |  |  |  |  |  |
| SAFETY |  |  |  |  |  |
|  | a. | Qualified Helibase Manager assigned. |  |  |  |
|  | b. | Helibase/Helispot meets established standards. |  |  |  |
|  | c. | Organizational chart posted. |  |  |  |
|  | d. | Communications plan posted. Frequency assignments known. |  |  |  |
|  | e. | Helibase/Helispot fire protection meets established standards. |  |  |  |
|  | f. | Crash rescue/evacuation kits on the Helibase/Helispot. |  |  |  |
|  | g. | Emergency plan posted/discussed. |  |  |  |
|  | h. | All personnel briefed and assignments know. |  |  |  |
|  | i. | Separation of aircraft provided (if more than one used). |  |  |  |
|  | j. | Personal protective equipment meets established standards. |  |  |  |
|  | k. | Air Hazard Map posted/hazards known to pilot(s). |  |  |  |
|  | l. | Weather considerations known/discussed. |  |  |  |
| AIRCRAFT/PILOTS |  |  |  |  |  |
|  | a. | Pilot and aircraft approved for mission. |  |  |  |
|  | b. | Pilot duty limitations status satisfactory. |  |  |  |
|  | c. | Load calculations prepared and posted. |  |  |  |
|  | d. | Aircraft radios functioning. |  |  |  |
|  | e. | Electrical/mechanical releases functioning. |  |  |  |
| DISPENSER |  |  |  |  |  |
|  | a. | Installation as instructed with restraints in place. |  |  |  |
|  | b. | Mechanical operation satisfactory. |  |  |  |
|  | c. | Extinguishing system filled and operational. |  |  |  |
|  | d. | Glycol reservoir filled and tightly capped. |  |  |  |
|  | e. | 20-second ignition delay achieved. |  |  |  |
|  | f. | Intercom operable. |  |  |  |
|  | g. | Pilot has been briefed and agrees that all is in order. |  |  |  |
|  | h. | Sphere container secured in place. |  |  |  |
|  | i. | Knife available for emergency use. |  |  |  |
|  | j. | Additional container of water available. |  |  |  |
| SUPPORT EQUIPMENT/PERSONNEL |  |  |  |  |  |
|  | a. | Adequate support equipment/personnel to complete mission. |  |  |  |
|  | b. | Pump/engine operational checks complete. |  |  |  |
|  | c. | Radios/communications operational check. |  |  |  |
|  | d. | Support equipment/personnel prepositions before actual operations begin. |  |  |  |
|  | e. | Adequate supply of plastic spheres and glycol to complete project. |  |  |  |
|  | f. | Plastic Sphere dispenser Checklist completed. |  |  |  |

Signature _____________________________________		Date ______________

JOB HAZARD ANALYSIS- Plastic Sphere Dispenser

| FS-6700-7 (8/96) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| United States Department of Agriculture |  | 1. WORK PROJECT/ACTIVITY |  |  | 2. LOCATION |
| FOREST SERVICE |  | Plastic Sphere Dispenser |  |  |  |
| JOB HAZARD ANALYSIS (JHA) |  | 4. NAME OF ANALYST |  |  | 5. JOB TITLE |
| References-FSH 6709.11 and .12 |  |  |  |  |  |
| 7. TASKS/PROCEDURES | 8. HAZARDS |  | 9. ABATEMENT ACTIONS |  |  |
|  |  |  | Engineering Controls * Substitution * Administrative Controls * PPE |  |  |
| Qualifications | Unqualified personnel |  | Sphere dispenser operator shall be certified annually. Pilot and helicopter will be certified annually for aerial ignition. Pilot will have two hours fire behavior and trained in use of the fire shelter. |  |  |
| Responsibilities | Unknown responsibilities |  | THe operator will be familiar with appropriate portions of IHOG and Interagency Aerial Ignition Guide. The project briefing will cover responsibilities and emergency procedures. |  |  |
| Bench testing | Possible malfunctions |  | Bench testing will be done prior to mounting the machine in the helicopter and will be done at a safe distance from the helicopter. |  |  |
| Personal Protective Equipment | Injuries |  | Operator and pilot shall have a fire shelter and wear flight helmet, nomex flight suit, nomex gloves and leather boots. The Burn Boss will be required to have the same PPE if riding in the helicopter. |  |  |
| Malfunctions | Possible fire in machine, machine not operating properly |  | Malfunctions will be addressed in the project briefing. Operator will immediately notify pilot of a malfunction and take appropriate action to correct it. If the malfunction cannot be corrected in the air, the helicopter will set down. If a fire occurs in the machine that the operator cannot extinguish, the pilot will be notified and the machine will be jettisoned. |  |  |
| Ignition | Firing pattern/ fire behavior |  | Firing pattern will be determined by the Burn Boss to ensure objectives are met safely. |  |  |
| Flight following | Loss of communication |  | Flight following will be done from the helibase during the project. Alternate flight following frequency listed on the 220 will be implemented if necessary. |  |  |
| 10. LINE OFFICER SIGNATURE | 11. TITLE |  |  | 12. DATE |  |