FIRE BEHAVIOR FORECAST

| INCIDENT: | # DRY LAKES COMPLEX & TURNBO |  | OPERATIONAL PERIOD: | 7/24/03 (day) |  |
| --- | --- | --- | --- | --- | --- |
| ADMINISTRATIVE UNIT: |  | # Gila NF |  | FORECAST#: |  |
| WEATHER SUMMARY: |  |  |  |  |  |
| The moisture that moved over the area yesterday will result in some lingering effects today. Relative humidities last night could be higher than stated in the spot forecast (65-80%), particularly on the east side of Dry Lake Fire. Daytime humidities should drop only to the mid-teens or higher with temperatures reaching into the low 80s at the higher elevations to high 90s at the lower elevations. Isolated thunderstorms should develop again this afternoon, although not as widespread as yesterday and more likely to be dry. Weak inversions should form about midnight and break by mid-morning. Transport winds will be southeast to southwest, but slope winds will predominate in the afternoon and continue to override the general winds. See attached spot fire weather forecast(s). |  |  |  |  |  |
| EXPECTED FIRE ACTIVITY |  |  |  |  |  |
| Short Range: Early morning relative humidities may reach as high as 100 percent under any inversions, limiting fire spread significantly until the inversion breaks. Expect more active burning after the inversion breaks until at least 2000. Spread rates will be slower than the past 2-3 days with flame lengths generally back to the less than 4-foot category and pinyon-juniper generally will not carry fire for the next day or two. However, flame lengths up to 6 feet and spread rates near 26 chains an hour may still happen in the afternoon on the hotter aspects. In the brush, spread rates will be near 35 chains an hour with 7-foot flame lengths. Probability of ignition still will be near 100% in the afternoon. Crowning is a possibility as slopes warm and upslope winds increase. 1- and 10-hour fuel moistures will be down to 2%. Live fuels will remain near 70% and will burn readily. Mid-Range: The high will strengthen and dry out the air mass once more through the weekend. Isolated thunderstorms will diminish each day and inversions will strengthen once more. ERCs at all stations are at record levels. The 1 and 10 hour fuel moistures will remain near 1% and 3% and temperatures near 100 at lower elevations. These conditions are expected to remain for the next six days days, at least. 100- and 1000-hour fuels are also very dry and have been consuming completely. Expect fire activity to pick up each day as the air mass dries: common spread of 10 chains an hour, and rapid upslope runs as the fire burns into heavier fuels with possible short crowning runs as flame lengths near 5 feet. The inversions will continue and fire activity will increase as the inversions break. Pinyon-juniper will begin to carry fire once more in the absence of further moisture. |  |  |  |  |  |
| SPECIFIC FIRE BEHAVIOR: |  |  |  |  |  |
| Dry Lake and Moonshine: Dry Lake will continue to spread primarily east and west above the breaks along the south side of the West Fork Gila River, west and north in the area between Turnbo Canyon and McKenna Park, and south through the Bloodgood Fire. Moonshine will continue to spread south and east above the breaks along the north side of the West Fork Gila River. Both fires will continue following the fuels. Fire activity will pick up after the inversion breaks with upslope runs possible in the afternoon. Slope winds will override general winds as slopes heat, particularly on south and west aspects. Fire behavior should be minimal early in the morning, especially where relative humidity exceeds 80 percent for more than one hour. Later in the day, expect backing fires, 2-5 chains an hour, and upslope runs up to 30 chains an hour or greater. Crown fire is possible on the hotter slopes, though less likely with the higher expected humidities. South slopes will exhibit the greatest fire activity due to radiant heating, lower humidity and stronger upslope winds. Watch for fires below the breaks to the West Fork Gila River if fire falls off the cliffs. Granny: Granny will continue to be the least active of the fires. Look for a spread of 10 chains an hour or less as the fire moves through down fuels. The side drainages have more fuel than the flats and spread will be greater as the fire spreads through the concentrations of fuel. |  |  |  |  |  |
| AIR OPERATIONS: |  |  |  |  |  |
| Thunderstorms may interfere with air operations in the afternoon, although storms are expected to be smaller and more isolated that yesterday. Look for the inversion to break about 1000 and smoke to clear by 1100. |  |  |  |  |  |
| SAFETY: |  |  |  |  |  |
| Don’t let the lower fire behavior lull you. Both live and dead fuel moistures remain extremely low. Take weather and watch for winds stronger than forecast and relative humidities lower than forecast. Be particularly alert for ember showers and widespread spotting from any torching or crowning runs to avoid entrapment. |  |  |  |  |  |

__________________________________				7/10/03		1900

Louisa Evers/LTAN							Date			Time