#### Fire Occurrence Areas

Geodataset Name:	FOA or XXXFOA where XXX is a three letter abbreviation for district

Geodataset Type:		Integer

Geodataset Feature:	GRID

DESCRIPTION:

The Fire Occurrence Area is based on historic fire ignition data from the Division Forestry, the USDI-Department of the Interior (DOI) and the Department of Defense (DOD).  Fire occurrence data for the DOI agencies included the National Park Service, the Fish and Wildlife Service and the Bureau of Indian Affairs.  Fire occurrence data for the DOD agencies included US Air Force’s Eglin Air Force Base.  Fire occurrence data was gathered for 20 years.  The years 1981-2000 are used for the base period.  For some agencies, fire occurrence data was not available for this entire time period and for these agencies, fire occurrence was annualized based on the number of years available.  A surface grid was generated from the point locations using the density modeling function available in ArcInfo GRID.

SOURCE DATA METADATA:

- Data source			DOF, USDI, USDA - FS, DOD

- Scale			N/A (most cases to nearest TRS)

- Data Format			Point

- Date			July 2001

- Geodataset extent		State of Florida

FINAL DATA METADATA:

- Data source			SIS

- Resolution			30m

- Data Format			Integer GRID

- Date			2002

- Geodataset extent		records cover the state of Florida

FEATURE ATTRIBUTE TABLES:

Datafile Name: FOA.STA

ITEM NAME	WIDTH	TYPE	PRECISION

MIN	15	NUMBER	3

MAX	15	NUMBER	3

MEAN	15 	NUMBER	3

STDV	15 	NUMBER	3

Datafile Name:	FOA.VAT

ITEM NAME 		WIDTH 		TYPE 		PRECISION

VALUE                     	10    		Binary

COUNT		            	10    		Binary

FEATURE ATTRIBUTE TABLES CODES AND VALUES:

Datafile Name: FOA.STA

ITEM	ITEM DESCRIPTION

MIN		Minimum value in dataset

MAX		Maximum value in dataset

MEAN		Mean value of dataset

STDV		Standard Deviation in dataset values

Datafile Name:	FOA.VAT

ITEM			ITEM DESCRIPTION

VALUE			(# fires / 1000 acres / year) * 1000

COUNT			Number of cells for each value

MAP PROJECTION PARAMETERS:

Projection			Albers

Datum				HPGN

Units				Meters

Spheroid			GRS1980

Parameters

First Standard Parallel 		24 00 00

Second Standard Parallel 		31 30 00

Central Meridian 			-84 00 00

Latitude of Origin 		24 00 00

False Easting (meters)          	400000

False Northing (meters)               	0

DATA SOURCE CONTACT(S):

Name:				Space Imaging Solutions

Abbr. Name:			SIS

Address:				Space Imaging Solutions

182 South 600 East, Suite 203

Salt Lake City, Utah 84102

Phone:				(801) 325 1006

Fax:				(801) 325 1009

Web Site:			www.spaceimaging.com

Contact Person:			Rebecca Megown

Contact E-mail:			rmegown@SpaceImaging.com