Silverspur Unit 7 Prep Specifications

The objective of the treatment is to create a solid holding line around the entirety of the unit perimeter.  This will be accomplished with chainsaw felling and bucking and hand followed by pile burning.

Project specifications:

- Silverspur Unit Prescribed Burn Unit:

  - Hand prep is authorized on BLM lands where cultural clearance has been verified, but no mechanized equipment has been authorized.

  - No known cultural avoidance sites exist within identified units.

  - The goal will be to treat vegetation within the interior of the planned holding line, generally located within the western and eastern most edges of the unit.

  - Removal of juniper or pine trees 3” or greater would be restricted from June 1-July 31 to protect bats. Use of hand crews may be employed between June 1-July 31 for removal of trees <3”dbh.  Trees may also be limbed during this restricted period.  Hand piling may also occur at any time.

    - Some prep was completed along Pine Tree Dr on the western edge of the unit during June and July last year.   If there are any dense concentrations of ponderosa between 3” and 8” those may be cut to the tree spacing below.

  - Notify Fuels Specialist if bird nests are discovered and avoid.  Raptor, eagle and Pinyon Jay nests may require up to 1 mile buffer.

  - Cut most live Ponderosa Pine <8” DBH and > 1’ tall while striving for a leave tree spacing (any size PP) of about 25-35’ between boles.

  - Cut all juniper.

  - Limb all leave trees up to 6 ft high (do not limb in excess of 1/3 of total canopy height).

  - Improve unit holding line/perimeter to accommodate type 6 engine holding and patrolling where feasible.

  - Fell standing dead snags within 100 feet of the holding line, and pile dead & down material within the treatment area.

  - All activity material plus existing dead and down material shall be hand piled with the following specifications:

- Piles should be placed > 25’ from fences, two-track roads, or trails or other improvements.

- Hand piles will be >6ft tall & compact.

- Pile out from the dripline of trees and away from large dead and down, snags, stumps and duff that could create smoldering and creeping.  Open grassy or bare areas make the best pile locations.

- Lop and scatter slash to a slash height less than 12” when a pile of sufficient size cannot be created from generated slash for a 200’ radius (i.e. in open grass land).  Scattered slash should not be windrowed and should generally be at least 100’ from holding lines.

- Piles may be located in the interior or exterior of the burn unit at least 30’ away from control line as long as the previous specifications are met.

Safety, Medical, Comm:

Fill out and follow INCIDENT/PROJECT MEDICAL EMERGENCY EVACUATION PLAN or MEEP

All helispots shown on the map have been approved to land up to a Type 2 helicopter.