| RESPONSE LEVEL | 1 | 2 | 3 |
| --- | --- | --- | --- |
| WR-BI | 0 – 23.9 | 24 – 33.9 | 34+ |
| Response | 1 Engine 1 Prevention | 2 Engines 1 Crew 1 Dozer 1 Air Attack 1 Prevention | 4 Engines 2 Crews 1 Water Tender 1 Dozer 1 Air attack 1 Helicopter 1 Rappel Load 1 Prevention 1 LEO |

| P R O C E D U R E S | Dispatch closest forces based on current dispatch level - See table above. Primary Notifications: District Duty Officer Forest Duty Officer Other Notifications (within 1 mile): Grants Pass ODF Medford BLM Special Considerations/Hazards/Values: Eight Dollar Botanical Area Illinois Wild and Scenic River Dip Sites See additional sites on spreadsheet Move Up |
| --- | --- |

|  |  |  |  |  |  | Aviation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Repeater | Rx | Tone | Tx | Tone |  | Name | Rx | Tone | Tx | Tone |
| Bolan | 171.3875 | 67.0 | 164.9625 | 203.5 |  | Air to Ground 62 | 169.3625 |  | 169.3625 |  |
| Onion | 171.3875 | 67.0 | 164.9625 | 179.9 |  | Air to Ground 51 | 168.3125 |  | 168.3125 |  |
| Fiddler | 171.3875 | 67.0 | 164.9625 | 186.2 |  | Nat’l Flight Follow | 168.6500 | 110.9 | 168.6500 | 110.9 |